3GPP TSG RAN WG4 Meeting #95-e                                                                 		R4-2009290

e-Meeting, May 25 - June 5, 2020

3GPP TSG RAN WG1 Meeting #100bis                                                                 	R1-2002908

e-Meeting, April 20 - April 30, 2020

Title:	LS on intra-cell guard band configuration for NR-U

Response to:

Release:	Rel-16

Work Item:	NR_unlic-Core

Source:	RAN WG1

To:	RAN WG2, RAN WG4

Cc:

Contact Person:

#### Name:	Seonwook Kim

E-mail Address:	seonwook.kim@lge.com

Attachments:	none

1. Description:

RAN1 has made the following agreements related to intra-cell guard band configuration for operation with shared spectrum channel access.

Agreement:

The intra-carrier guard bands on a carrier can be semi-statically adjusted with an RB level granularity. The RAN4 minimum guard band requirements are used as the guard bands when no semi-static adjustment is applied.

- The guard bands adjustments do not affect the already agreed restrictions on PUCCH resource allocation.

Agreement:

UE determines the number of RB sets (corresponding to LBT bandwidths) and the available PRBs in each RB set, both for DL and UL, based on,

- If configured, the RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 (if UL is configured) configure the lists of intra-carrier guard-bands per cell, e.g.,

  - If intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 are given as {GB1low, GB1high}, …, {GBN-1low, GBN-1high} where GB is given by CRB index, the UE determines

    - The number of RB sets for a cell: N

    - The available PRBs in each RB set: [RBstart, GB1low-1] for RB set #1, [GB1high+1, GB2low-1] for RB set #2,…, [GBN-1high+1, RBend] for RB set#N, where RBstart and RBend corresponds to starting and ending RB index of cell, respectively.

    - Note: {GB1low, GB1high}, …, {GBN-1low, GBN-1high} may be provided separately for DL and UL

- If intraCellGuardBandDL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- If intraCellGuardBandUL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- Note: This addresses the FFS in sections 5.1.2.2 and 6.1.2.2 in 38.214.

Agreement

- The RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 include a mechanism to indicate that no intra-carrier guard-bands are configured

  - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP

RAN1 kindly asks RAN2 and RAN4 to take the above agreements on intra-cell guard band configuration into account for developing their specifications for operation with shared spectrum channel access.

2. Actions:

To RAN WG2 and WG4

ACTION: RAN1 would like to respectfully ask RAN2 and RAN4 to take the above information into account for developing their specifications.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #101-e	25 May - 6 June 2020, 		Online

TSG-RAN WG1 Meeting #102	24 - 28 Aug. 2020, 		Toulouse, FR