3GPP TSG-WG RAN4 Meeting #95-e	R4-2006595

# Online, 25th May – 5th June, 2020

Source: 		Nokia, Nokia, Shanghai Bell, [AT&T]

Title: 			TP to TR 36.716-03-02 on 3DL/2UL CA_2-14-30

Agenda Item: 		5.6.3 [LTE_CA_R16_xBDL_2BUL-Core]

Document for:		Approval

# Introduction

This TP introduces the following 3DL/2UL CA configurations to TR 36.716-03-02.

- 3BDL_2A-14A-30A_2BUL_2A-14A_BCS0

- 3BDL_2A-14A-30A_2BUL_14A-30A_BCS0

## TP to TR 36.716-03-02

## <Start of Changes>

## 6.X LTE-A inter-band CA: Band 2 and Band 14 and Band 30 DL with 2 bands UL

### 6.X.1	List of specific combination issues

#### 6.X.1.1	Channel bandwidth per operating band for CA

Table 6.X.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-14A-30A | CA_2A-14A CA_14A-30A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 14 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |

#### 6.X.1.2	Co-existence studies for LTE-A inter-band CA 3 bands DL CA_2A-14A-30A and 2 bands UL

For 2UL band 2 and band 14, the harmonics and intermodulation products are calculated in the following table.

Table 6.X.1.2-1: Co-existence study for DL CA_2A-14A-30A with UL CA_2A-14A

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 788 | 798 | 1850 | 1910 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1576 | 1596 | 3700 | 3820 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2364 | 2394 | 5550 | 5730 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1052 | 1122 | 2638 | 2708 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -334 | -254 | 2902 | 3032 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3426 | 3506 | 4488 | 4618 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 454 | 544 | 4752 | 4942 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4214 | 4304 | 6338 | 6528 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -2244 | -2104 | 5276 | 5416 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -6852 | -6602 | -1342 | -1242 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8188 | 8438 | 5002 | 5102 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4154 | -3954 | 1306 | 1456 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 7126 | 7326 | 6064 | 6214 |

For 2UL band 14 and band 30, the harmonics and intermodulation products are calculated in the following table.

Table 6.X.1.2-1: Co-existence study for DL CA_2A-14A-30A with UL CA_14A-30A

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 788 | 798 | 2305 | 2315 |
| 2nd harmonic | 2* f1_low | 2*f1_high | 2*f2_low | 2*f2_high |
| harmonic frequency limit (MHz) | 1576 | 1596 | 4610 | 4630 |
| 3rd harmonic | 3* f1_low | 3*f1_high | 3*f2_low | 3*f2_high |
| harmonic frequency limit (MHz) | 2364 | 2394 | 6915 | 6945 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1507 | 1527 | 3093 | 3113 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | -739 | -709 | 3812 | 3842 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3881 | 3911 | 5398 | 5428 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 49 | 89 | 6117 | 6157 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4669 | 4709 | 7703 | 7743 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -3054 | -3014 | 6186 | 6226 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -8472 | -8422 | -887 | -837 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10008 | 10058 | 5457 | 5507 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -5369 | -5319 | 2216 | 2266 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8491 | 8541 | 6974 | 7024 |

It is concluded that there is no harmonic or intermodulation relation to the own receiver bands in this 3DL/2UL band combination.

#### 6.X.1.3	MSD

No MSD issues are identified for this band combination.

#### 6.X.1.4	∆TIB and ∆RIB values

The relaxation values are already specified for 3DL/1UL in the following and is applied to 3DL/2U.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-14-30 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.5 |

Table 6.X.4-2: ΔRIB

| Inter-band CA Configuration | 2 | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-14-30 | 14 | 0.3 |
|  | 30 | 0 |
|  | 2 | 0.3 |

## <End of Changes>