3GPP TSG-WG RAN4 Meeting #95-e	R4-2006598

# Online, 25th May – 5th June, 2020

Source: 				Nokia, Nokia, Shanghai Bell, [AT&T]

Title: 					TP to TR 36.716-03-02 on 4DL/2UL CA_2-14-30-66

Agenda Item: 		5.6.2 [LTE_CA_R16_xBDL_2BUL-Core]

Document for:		Approval

# Introduction

This TP introduces the following 4DL/2UL CA configurations to TR 36.716-03-02.

- 4BDL_2A-14A-30A-66A_2BUL_2A-14A_BCS0

- 4BDL_2A-14A-30A-66A_2BUL_14A-30A_BCS0

- 4BDL_2A-14A-30A-66A_2BUL_14A-66A_BCS0

- 4BDL_2A-2A-14A-30A-66A_2BUL_2A-14A_BCS0

- 4BDL_2A-2A-14A-30A-66A_2BUL_14A-30A_BCS0

- 4BDL_2A-2A-14A-30A-66A_2BUL_14A-66A_BCS0

- 4BDL_2A-14A-30A-66A-66A_2BUL_2A-14A_BCS0

- 4BDL_2A-14A-30A-66A-66A_2BUL_14A-30A_BCS0

- 4BDL_2A-14A-30A-66A-66A_2BUL_14A-66A_BCS0

TP to TR 36.716-03-02

<Start of Changes>

## 6.X LTE-A inter-band CA: Band 2 and Band 14 and Band 30 and Band 66 DL with 2 bands UL

### 6.X.1	List of specific combination issues

#### 6.X.1.1	Channel bandwidth per operating band for CA

Table 6.X.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-14A-30A-66A | CA_2A-14A CA_14A-30A CA_14A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 14 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-2A-14A-30A-66A | CA_2A-14A CA_14A-30A CA_14A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 14 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-14A-30A-66A-66A | CA_2A-14A CA_14A-30A CA_14A-66A | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 14 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |

#### 6.X.1.2	Co-existence studies for LTE-A inter-band CA 4 bands DL CA_2A-14A-30A-66A and 2 bands UL

From the lower order fallback CAs, CA 2A-14A-30A, CA 2A-14A-66A, and CA 14A-30A-66A, the IMD issues are identified in the following.

- For 2 uplink CA_2A-14A, there is 4th IMD to DL band 66.

- For 2 uplink CA_14A-66A, there is 4th IMD to DL band 2.

#### 6.X.1.3	MSD

The requirements of low order combinations CA_3BDL_2A-14A-66A_2BUL_2A-14A and CA_3BDL_2A-14A-66A_2BUL_14A-66A can be applied.

#### 6.X.1.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below.

Table 6.X.1.4-1: ΔTIB,c

| E-UTRA CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-14-30-66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |

Table 6.X.1.4-2: ΔRIB,c

| E-UTRA CA Configuration | E-UTRA NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-14-30-66 | 2 | 0.4 |
|  | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |

<End of Changes>