3GPP TSG-RAN WG4 Meeting # 95-e	R4-2008965

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:			10.2

Source:	Moderator (Huawei)

Title:	Email discussion summary for [95e][137] NR_n13

Document for:	Information

Introduction

The scope of this email discussion is to discuss the contributions submitted at agenda 10.2 to specify a new NR FDD operating band n13.

The target of 1st round is to discuss the potential agreements on requirements for new NR band n13.

Topic #1: UE RF

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2008209 | Qualcomm Incorporated | The contribution presents simulations and measurements for NR band n13. Proposal 1: Use AMPR as shown in Section 2.3 of the contribution. |
| R4-2007311 | Huawei, HiSilicon | The contribution provides the draft CR to TS 38.101-1 on introduction of NR band n13. The RF requirements for n13 are added in relevant clauses. The A-MPR is still TBD since the discussion on A-MPR is ongoing. Moderator: It is a late submission and submitted for comments. |

Open issues summary

Sub-topic 1-1: A-MPR for NS_07

Issue 1-1:

- Proposals

  - Use the A-MPR proposed in R4-2008209

- Table 3: A-MPR regions for NS_07

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR | Meas. A-MPR DFT/CP |
| --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 |  |  |  |  |
|  |  | >0 | ≥ 1.8 | A3 |  |
|  |  | ≤ 0.9 | ≤ 0.36 | A4 |  |
| 5 MHz | 779.5 ≤ Fc < 782 |  |  |  |  |
|  |  | ≤ 0.9 | ≥ 0 | A1 |  |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |  |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |  |
|  |  | >3.42 | ≤ 0.36 | A4 |  |
| 10 MHz | Fc = 782 |  |  |  |  |
|  |  | ≤ 2.34 | ≥ 0 | A1 |  |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |  |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |  |
|  |  | > 6.48 | ≤ 0.36 | A4 |  |

- Table 4: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | [12] | [9] | [5.5] | [4] |
| DFT-s-OFDM QPSK | [12] | [9] | [5.5] | [4] |
| DFT-s-OFDM 16 QAM | [12] | [9] | [5.5] | [4] |
| DFT-s-OFDM 64 QAM | [12] | [9] | [5.5] | [4] |
| DFT-s-OFDM 256 QAM | [12] | [9] | [5.5] | [4] |
| CP-OFDM QPSK | [13] | [10] | [6.5] | [4] |
| CP-OFDM 16 QAM | [13] | [10] | [6.5] | [4] |
| CP-OFDM 64 QAM | [13] | [10] | [6.5] | [4] |
| CP-OFDM 256 QAM | [13] | [10] | [6.5] | [4] |

- Recommended WF

  - Further discussion and wait for more simulations and measurements

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Qualcomm | Sub topic 1-1: Adding some measurements for the record. Highlighted. Channel Bandwidth, MHz; Carrier Frequency, MHz; Regions; A-MPR; Meas. A-MPR DFT/CP. RBstart*12*SCS MHz; LCRB*12*SCS MHz. 5 MHz; 782 ≤ Fc ≤ 784.5. >0; ≥ 1.8; A3. ≤ 0.9; ≤ 0.36; A4. 5 MHz; 779.5 ≤ Fc < 782. ≤ 0.9; ≥ 0; A1; 12/12. > 0.9, ≤ 1.26; ≥1.26; A2; 8/9. > 1.26, ≤ 3.42; ≥ 1.8; A3; 4.5/6. >3.42; ≤ 0.36; A4; analysis. 10 MHz; Fc = 782. ≤ 2.34; ≥ 0; A1; 12/12. >2.34, ≤ 3.24; ≥ 1.44; A2; 8/9. > 3.24, ≤ 6.48; ≥ 3.24; A3; 4.5/6. > 6.48; ≤ 0.36; A4; analysis. |
| Company B | Sub topic 1-1: |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007311 | Company A: |
|  | Company B: |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1: A-MPR for NS_07 | Qualcomm made an update on measurement values that follows the simulation results. Moderator suggestion: wait for more simulations and measurements in next meeting. R4-2008209 is to be Noted |

TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2007311 | It is a late submission and submitted for comments To be Noted |

Discussion on 2nd round (if applicable)

None

Topic #2: BS RF requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007312 | Huawei, HiSilicon | The contribution provides the draft CR to TS 38.104 on introduction of NR band n13. The requirements for n13 are added in relevant clauses. |

Companies views’ collection for 1st round

CRs/TPs comments collection

| number | Comments collection |
| --- | --- |
| R4-2007312 | Nokia: In Table 6.6.5.2.3-6, “13” shall be changed to “n13” |
|  | Huawei: agree with Nokia comment, it can be updated in the revision. |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2007312 | To be revised |

Discussion on 2nd round (if applicable)

CRs/TPs comments collection

| number | Comments collection |
| --- | --- |
| R4-2008923 （revision of R4-2007312） |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |