3GPP TSG-RAN WG4 Meeting #95-e	R4-200xxxx

Online, May 25 – June 5, 2020

Agenda item:	6.8.2.1.5

Source: 	Qualcomm Incorporated

Title: 	Link-level simulation assumptions for UE Rx-Tx time difference measurements

Document for:	Discussion

# Introduction

In RAN4#94-e-Bis meeting, it was agreed to bring forth simulation assumptions for UE Rx-Tx time difference measurements [1]:

Link-level simulation plan for UE Rx-Tx time difference:

- Link-level simulations plan:

  - RAN4#95 – try to agree on simulation assumptions,

  - RAN4#96 – provide first simulation results.

In this paper, we present simulation assumptions for UE Rx-Tx time difference measurement. Some of the assumptions are common to RSTD/RSRP link-level simulation assumptions [2].

# Simulation assumptions

Table 1: General parameters

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 cells at distinct locations: <cell 1, cell 2, cell3>, where cell 1 is the serving/reference cellNOTE 1 |  |
|  | • |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH transmissions in the measured cell: | no other cell transmissions in its positioning symbols, except PRS - non-PRS symbols: 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 10 MHz, 20 MHz, 50 MHz - 15 kHz - FDD, TDD - 4 GHz - 20 MHz, 50 MHz, 100 MHz - 30 kHz - FDD, TDD | - 40 GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, TDL-C (300 ns delay spread, 100 Hz), TDL-A (30 ns delay spread, 5Hz), TDL-B (100 ns delay spread, 200Hz) | AWGN, TDL-C (60 ns delay spread, 300 Hz) |
| Es/Iot for tw cells (cell 1, cell 2), [dB] | (-3, -6, -13) | (-3, -6, -13) (-3, -6, -10) |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |
| NOTE 1: nr-DL-PRS-ReferenceInfo is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). |  |  |

Table 2: PRS transmission configuration parameters

| Parameter | Value |  |
| --- | --- | --- |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: - 52 (10MHz), 104 (20MHz), 268 (50MHz) - 30 kHz: - 48 (20MHz),132 (50MHz), 272 (100MHz) | - 120 kHz: - 32(50MHz),64(100MHz), 128 (200MHz) |
| Comb | Comb-6, comb-2 |  |
| DL-PRS-ResourceRepetitionFactor | 1, 2, 4, 16 |  |
| PRS-ResourceTimeGap | 1, 2, 4 | 1, 8, 16 |
| PRS muting | No muting, muting (comb-2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 1 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| NOTE 1: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). |  |  |

At least the following performance characteristics are to be provided for TUE-RX:

- TUE-RX error CDFs for the 3 cells

- 90%-ile of the TUE-RX errors for each cell

In the above,

- TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).

Companies are encouraged to identify the number of samples used for deriving TUE-RX error.

# References

[1] R4-2005378

[2] R4-2002287