3GPP TSG-RAN WG4 Meeting #95-e 	DRAFT R4-2008860

Online, 25 May - 05 Jun 2020

Source: 	Huawei

Title: 	OTA BS testing Rx FR2 MU calculation tables

Agenda Item:	6.19.5

Document for:	Approval

# Introduction

This is  resubmission of the RX FR2 MU calculation tables R4-2004532,  the tables were submitted to RAN4#94bis-e but were not approved the TE companies wished to confirm the TE MU values used.

In particular the value for the IAC in the frequency range 37<f<40GHz is 0.06 dB larger than the previously agreed MU value. It was requested to have another meeting cycle to try to resolve this minor issue (it is not intended to change any agreed MU or TT values only resolve eth MU calculation table)

# Discussion

As with FR1 the Rx MU calculations are based on the EIS measurement, this is then used to derive the other MU values for the other FR2 RX tests where appropriate.

The MU calculation tables have been taken from TR 37.817-02 and consolidated in a spreadsheet.

On inspection there are a number of issues with the FR2 Rx calculations which were not present in any of the Tx or the FR1 Rx calculations, primarily:

- Test equipment uncertainty is not consistent, and does not seem to be agreed

- There are 3 different proposals for the CATR with very different error values.

In the spreadsheet I have attempted to resolve these errors, using a number of techniques:

- using similar values previously agreed for the TX for example:

    - -	 NA and SGH are used in both Tx and Rx

    - -	Misalignment errors, mismatch errors etc that are common to Tx and Rx

- If no other guidance available taking the average of the 3 companies proposals.

The action taken in each case is highlighted in the spreadsheet

The final values are in line with the agreed MU values (which we do not intend to change).

# 3	Spreadsheet construction

Summary sheet

A summary sheet with the final MU values for each of the requirements for each of the OTA chambers.

The final 2 column indicate if one or more of the chamber MU values exceed the agreed common maximum accepted test system uncertainty.

|  | Expanded uncertainty [dB] |  |  |  |  |  | A method exceeds agreed value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IAC |  | CATR |  | Agreed value |  |  |  |
|  | 24.25<f<29.5GHz | 37<f<40GHz | 24.25<f<29.5GHz | 37<f<40GHz | 24.25<f<29.5GHz | 37<f<40GHz | 24.25<f<29.5GHz | 37<f<40GHz |
| EIS | 2.33 | 2.46 | 2.25 | 2.33 | 2.40 | 2.40 |  | x |

TE sheet

This sheet contains the MU values for the test equipment and the conducted MU values (minus mismatch) which are used in all the chamber calculations.

The all use of these numbers is referenced to this sheet.

As there previously no agreed values I have selected a proposed value and given an explanation as to why.

Chamber Error sheets

The following 2 sheets:

CATR-Er

IA-Er

Contain the errors and MU values for the different chamber types (as indicted), these errors are referenced throughout the different requirements to ensure that for the same error the same value is used in each of the MU budgets.

The IAC has 1 change for the uncertainty of the signal generator. The value selected on the TE page is higher than the one used, the value choosen was the one supplied by Keysight as they are the TE vendors who contributed. The result is the final values re higher but still less than the previously agreed value.

The CATR has many discussion points are there are 3 separate budgets calculated which I have attempted to consolidate, the reasoning is captured in the spreadsheet. The final values of 2.25/2.33 are less than the previously agreed MU values.

Requirement MU calculation sheets

There is only one requirement calculated EIS.