3GPP TSG RAN WG4 Meeting #95-e	R4-2008341

Online, 25th May – 5th June 2020

Agenda item:	8.3.2

Source:	SoftBank Corp., LG Electronics Inc.

Title:	TP update for TR 37.716-11-11: DC_11_n28

Document for:	Approval

1	Introduction

2B EN-DC of DC_11_n28 was approved in RAN4#94 [1] but REFSENS exceptions have TBD parts. This TP is to update the REFSENS exception results for 4th IMD and reflect the REFSENS exception considering the spectrum allocation in Japan since B11 is the specific band used in only Japan..

2	Text Proposal

[Unchanged Parts Skipped]

### 6.1.86		DC_11_n28

#### 6.1.86.1		Operating bands for DC

Table 6.1.86.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_11_n28 | 11 | n28 | No |

#### 6.1.86.2		Configuration for DC

Table 6.1.86.2-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_11A_n28A | DC_11A_n28A | 11A | n28A |

#### 6.1.86.3		Co-existence studies

Table 6.1.86.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_11A_n28A | 23 | +2/-3 |

#### 6.1.86.4		Spurious emission band UE co-existence for DC

For DC_11_n28, the TIB,c and RIB,c values for UEs not supporting simultaneous Rx/Tx are given in the tables below.

Table 6.1.86.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11_28 | E-UTRA Band 3, 18, 19, 34 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 1, 42, 65 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 9 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 9: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. |  |  |  |  |  |  |  |

#### 6.1.86.5		MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.86.5-1

Table 6.1.86.5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 5711.6 | 5791.6 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 7139.5 | 7239.5 | 3515 | 3740 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 8567.4 | 8687.4 | 4218 | 4488 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 9995.3 | 10135.3 | 4921 | 5236 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 679.9 | 744.9 | 2130.9 | 2195.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2107.8 | 2192.8 | 41.9 | 68.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3558.8 | 3643.8 | 2833.9 | 2943.9 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3535.7 | 3640.7 | 661.1 | 816.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1359.8 | 1489.8 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4986.7 | 5091.7 | 3536.9 | 3691.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4261.8 | 4391.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1564.1 | 1364.1 | 5088.6 | 4963.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 611.8 | 786.8 | 2937.7 | 2787.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4239.9 | 4439.9 | 6414.6 | 6539.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4964.8 | 5139.8 | 5689.7 | 5839.7 |

Based on Table 6.1.86.5-1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 11, 21, 32, 45, 50, 51, 74, 75, 76, , n91, n92, n93 and n94.

- 3rd order harmonic may fall into Rx frequencies of band 1, 4, 10, 23, 65 and 66.

- 2nd order IMD products may fall into Rx frequencies of bands 1, 4, 10, 12, 17, 23, 29, 44, 65, 66, 67 and 85.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 4, 10, 22, 23, 42, 43, 48, 49, 65, 66, n77 and n78.

- 4th order IMD products may fall into Rx frequencies of bands 11, 12, 13, 14, 17, 20, 22, 28, 29, 32, 42, 43, 44, 45, 48, 49, 50, 51, 67, 68, 74, 75, 76, 85, n77, n78, n79, n91, n92, n93 and n94.

- 5th order IMD products may fall into Rx frequencies of bands 11, 12, 13, 14, 17, 21, 24, 28, 32, 44, 45, 50, 51, 67, 68, 71, 74, 75, 76, 85, n79 n91, n92, n93 and n94.

Based on above,

- the 2nd order harmonic of Band n28 will fall into Band 11.

- the 4th and 5th order IMD may fall into Rx frequencies of band 11 and n28.

Regarding the MSD due to harmonics of Band 28 fall into Band 11, the exceptions are needed. Based on CA_11-28, reference sensitivity exceptions for DC_11_n28 due to UL harmonic are proposed in Table 6.1.86.5-2 with uplink configuration specified in Table 6.1.86.5-3.

Table 6.1.86.5-2: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 112,10,13 | 24.8 | 21.8 |  |  |  |  |  |  |  |  |  |  |
| NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n77, DC_66_n77, DC_3_n77, DC_3_n78 and DC_11_n28 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.86.5-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | 11 | 12 | 25 |  |  |  |  |  |  |  |  |  |  |

Regarding the MSD due to 4th and 5th order IMD, the exceptions are needed. Band 11 is the specific band used in only Japan. Considering the spectrum allocation in Japan, only the MSD analysis for n28 Rx is needed. The reference sensitivity exceptions for DC_11_n28 due to IMD are proposed in Table 6-1.x. 5-4.

Table 6.1.86.5-4: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A-n28A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 10.4 | FDD | IMD4 |

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.86.5-5 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.86.5-5: Band 11 and Band n28 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | 5th IMD |
| Galileo | 1559 | - | 1591 | Yes |  | 5th IMD |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | 5th IMD |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | No | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 5th IMD |
|  | 5150 | - | 5350 | Yes | Europe | 5th IMD |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 5th IMD |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 |  | USA Canada |  |
|  | 57000 | - | 64000 |  | South Korea |  |
|  | 59000 | - | 66000 |  | Japan |  |
|  | 59000 | - | 64000 |  | China |  |
|  | 59400 | - | 62900 |  | Australia |  |

#### 6.1.86.6	∆TIB and ∆RIB values

For DC_11_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.86.1.6-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_11_n28 | 11 | 0.4 |
|  | n28 | 0.6 |

Table 6.86.1.6-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_11_n28 | 11 | 0 |
|  | n28 | 0.2 |

#### 6.1.86.7		Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL inter-modulation products (IMD), or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.x.5 except for harmonic mixing and UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.86.7-1.

Table 6.1.86.7-1: Band 11 and Band n28 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_11A_n28A | 11 | a | 0 | b | -1 | 2 | 11 | Harmonic |
|  | n28 | c | 2 | d | 0 |  |  |  |
| DC_11A_n28A | 11 | a | 2 | b | -1 | 4 | 11 | IMD |
|  | n28 | c | -2 | d | 0 |  |  |  |
| DC_11A_n28A | 11 | a | -1 | b | 0 | 4 | n28 | IMD |
|  | n28 | c | 3 | d | -1 |  |  |  |
| DC_11A_n28A | 11 | a | 1 | b | -1 | 5 | 11 | IMD |
|  | n28 | c | -4 | d | 0 |  |  |  |
| DC_11A_n28A | 11 | a | 2 | b | 0 | 5 | n28 | IMD |
|  | n28 | c | -3 | d | -1 |  |  |  |

Equations (1) and (2) below are used to calculate the interference center frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 7-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

[Unchanged Parts Skipped]

3.  Reference

[1]  R4-2002599  TP for TR 37.716-11-11: DC_11_n28