3GPP TSG-RAN WG4 Meeting #95-e	R4-2008811

Electronic Meeting, 25th May – 5th June, 2020

Title: 	Simulation assumptions for NR URLLC BS performance requirements test cases

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2

Document for:	Approval

Introduction

This document provides the summary of the simulation assumptions for the NR URLLC BS performance requirements alignment for the RAN4 #96-e meeting, the simulation assumptions are based on the agreements from [1], [2] and [3]:

Simulation assumptions

- PUSCH demodulation requirements for high reliability with higher BLER

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2S:2U 30 kHz SCS: 7D1S2U, S=6D:4S:4U |
| Antenna configuration |  | 1x2, Low |
| Number of layers |  | 1 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0,2,3,1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Starting symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 for 30 kHz SCS FFS 15 kHz SCS |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hoping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Propagation condition and correlation matrix |  | TDLB100-400 Low |
| Testing metric |  | Target BLER: 10-2 (Calculate the target BLER after all transmission) |

- PUSCH demodulation requirements for PUSCH mapping Type B

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2S:2U 30 kHz SCS: 7D1S2U, S=6D:4S:4U |
| Antenna configuration |  | 1x2, Low |
| Number of layers |  | 1 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | 0 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Starting symbol | 0 |
|  | Allocation length | 2 |
|  | PUSCH aggregation factor | 1 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hoping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Propagation condition and correlation matrix |  | TDLC300-100 Low |
| Testing metric |  | 70% maximum throughput |

FRC

- PUSCH demodulation requirements for high reliability with higher BLER

| Reference channel | G-FR1-A3-xx | G-FR1-A3-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 30 |
| Allocated resource blocks | 52 | 106 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |
| Modulation | QPSK | QPSK |
| Code rate (Note 2) | 99/1024 | 99/1024 |
| Payload size (bits) | 1608 | 3240 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 1624 | 3256 |
| Total number of bits per slot | 16224 | 33072 |
| Total symbols per slot | 8112 | 16536 |

- PUSCH demodulation requirements for PUSCH mapping Type B

| Reference channel | G-FR1-A3-xx | G-FR1-A3-xx |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 30 |
| Allocated resource blocks | 52 | 106 |
| CP-OFDM Symbols per slot (Note 1) | 1 | 1 |
| Modulation | QPSK | QPSK |
| Code rate (Note 2) | 308/1024 | 308/1024 |
| Payload size (bits) | 576 | 1160 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 592 | 1176 |
| Total number of bits per slot | 1872 | 3816 |
| Total symbols per slot | 936 | 1908 |