3GPP TSG RAN WG 4 Meeting #95-e		TDoc R4-2006124

Electronic Meeting, 25 May - 5 June, 2020

3GPP TSG RAN WG 1 Meeting #100bis-e		TDoc R1-2002992

Electronic Meeting, 20 - 30 April, 2020

Title:	LS on removing the word “candidate” in Subclause 8.1.1 of TS 38.133 v16.3.0

Response to:

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN1

To:	RAN4

Cc:

Contact person:	Hongbo Si,

Hongbo_dot_Si_at_samsung_dot_com,

Amitav Mukherjee,

Amitav_dot_Mukherjee_at_charter_dot_com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN WG1 has agreed to utilize two sets of indexes for SS/PBCH block in Rel-16, i.e., SS/PBCH block index and candidate SS/PBCH block index. For operation without shared spectrum channel access, the two indexes are the same. For operation with shared spectrum channel access, a candidate SS/PBCH block index is utilized for indexing the candidate SS/PBCH blocks in a half frame, and the UE can determine an SS/PBCH block index according to , or according to , where  is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block,  is the candidate SS/PBCH block index, and  is either provided by ssbPositionQCL-Relationship-r16 or, if ssbPositionQCL-Relationship-r16 is not provided, obtained from a MIB provided by a SS/PBCH block with .

According to above, RAN WG1 expects a corresponding change of the index of SS/PBCH block in the Subclause 8.1.1 of TS 38.133 v16.3.0, and RAN WG1 kindly asks RAN WG4 to take the following TP into account for developing their specification. Note that for operation without shared spectrum channel access, a candidate SS/PBCH block index is the same as an SS/PBCH block index, hence, the change does not affect the operation without shared spectrum channel access, and is applicable to both operations with and without shared spectrum channel access.

====================== Start of TP for Subclause 8.1.1 of TS 38.133 v16.3.0 ===================

================================== Text Omitted =====================================

UE shall be able to monitor up to NRLM RLM-RS resources of the same or different types in each corresponding carrier frequency range, depending on a maximum number Lmax of candidate SSBs per half frame according to TS 38.213 [3], where NRLM is specified in Table 8.1.1-2, and meet the requirements as specified in clause 8.1. UE is not required to meet the requirements in clause 8.1 if RLM-RS is not configured and no TCI state for PDCCH is activated.

================================== Text Omitted =====================================

====================== End of TP for Subclause 8.1.1 of TS 38.133 v16.3.0 ====================

# 2	Actions

To RAN4:

ACTION: 	RAN1 kindly asks RAN4 to take above TP into account for developing their specification.

# 3	Dates of next TSG RAN WG 1 meetings

TSG-RAN WG1 Meeting #101-e		25 May – 5 June 2020, 		e-Meeting

TSG-RAN WG1 Meeting #102		24 – 28 August 2020, 		Toulouse, France