3GPP TSG-RAN WG4 Meeting #96-e		R4-2009728

E-meeting, 17 – 28 August, 2020

Agenda item:	7.10.3.1

Source:	Intel Corporation

Title:	Discussion on UE demodulation requirements for FR2 DL 256QAM

Document for:	Discussion

# Introduction

In the previous meeting WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM was agreed [1].

In this paper we provide our view on UE demodulation requirements for FR2 256QAM.

# Discussion

In the previous RAN4 meeting, the following agreements were reached on demodulation requirements:

- Tx EVM - 3% - Rx impairment modelling and band agnostic requirements - The following scenarios can be considered for band agnostic requirements definition; and FFS for other scenarios - Static channel mode: MCS 20-23, rank 1 - TDL-D channel mode: MCS 20-21, rank 1 - TDL-A channel mode: MCS 20-21, rank 1 - FFS on whether to explicitly model Rx impairment until the next meeting - Rank - Option 1: Rank 1 - Option 2: Define requirements for Rank 1, FFS on whether to define requirements for rank 2 - Option 3: Define requirements for single Rank configuration, i.e., either rank1 or rank 2 - Channel bandwidth and PRB allocation - 50MHz CBW with full PRB allocation - Propagation condition - Option 1: Fading channel - Option 1A: TDLA30-300 - Option 1B: TDLD30-75 - Option 1C: TDLD30-35 - Option 2: Static channel - TBD in the next meeting based on more companies’ simulation results. - MCS - MCS 20 for rank 1, if requirements for rank 1 are to be defined. - FFS for rank 2 - MIMO configuration - Depend on the agreement on propagation condition. Use 2Tx 2Rx ULA low if it is agreed to use fading channel, and use 1Tx 2Rx if it is agreed to use static channel.

Taking into account that FR2 requirements are limited by testable SNR, MCS and Rank should be defined to ensure testability under different test equipment conditions. In Table 1 we provide the maximum SNR values from TR 38.810 [2] for DFF and IFF methods.

Table 1. Max testable SNR in FR2.

| Channel bandwidth | Test method | Max SNR |
| --- | --- | --- |
| 100 MHz | DFF | 19.4dB |
|  | IFF | 19.7dB |
| 200 MHz | DFF | 16.4dB |
|  | IFF | 16.7dB |

From Table 1 we can observe that SNR depends on test method. For analysis we suggest to assume test method with the worst SNR condition (i.e. DFF), because test method is up to TE decision and one of these methods cannot be selected as a baseline. For DFF method and CBW 100 MHz, maximum testable SNR is 19.4 dB. Same time, based on agreement from previous meeting, 50 MHz CBW will be used for FR2 256QAM requirements. Therefore, SNR 22.4 dB can be assumed as maximum testable for further analysis.

Table 2 provides list of parameters used for link level analysis.

Table 2. Link level simulation assumptions.

| Parameter | Value |
| --- | --- |
| CBW | 50 MHz |
| SCS | 120kHz |
| Allocated RBs | Full allocation |
| Propagation | Static (AWGN) TDL-D 30ns delay spread, 35Hz Doppler frequency TDL-A 30ns delay spread, 300Hz Doppler frequency |
| MCS | MCS 20 in TS 38.214 Table 5.1.3.1-2 |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPT-RS: 2 (every 2 RBs), LPT-RS: 1 (every 1 symbol) |
| Impairments | TX: EVM = 3% RX: No (Ideal) |

In Table 3 summary of link level simulation results is presented.

Table 3. PDSCH performance

| Channel model | SNR for 70% of max T-put |  |
| --- | --- | --- |
|  | Rank 1 | Rank 2 |
| Static | 16.1 | 19.1 |
| TDL-D | 17.9 | 23.9 |
| TDL-A | 18.9 | 29.5 |

Based on PDSCH simulation results collection for Rel-15 NR WI, we can observe that difference of ideal and impairment results for FR1 256QAM and FR2 64QAM is about 2.5 – 3 dB. Based on this, we will have the following limitation on SNR for ideal results equal to 19.4 dB. Based on such limitation we can observe that we have sufficient margin only for scenarios with Rank 1 transmission and Static or TDL-D channel model.

Proposal 1:	Define FR2 256QAM demodulation requirements only for scenarios with Rank 1 and Static or TDL-D channel model.

# Conclusion

In this paper we provided view on FR2 256QAM demodulation requirements definition and made the following proposals:

Proposal 1:	Define FR2 256QAM demodulation requirements only for scenarios with Rank 1 and Static or TDL-D channel model.

# References

- R4-2008817 “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom, RAN4 #95-e, May 2020.

- TR 38.810 “Study on test methods”