3GPP TSG-RAN4 Meeting #96-e 		R4-2009761

Electronic meeting, 17th Aug – 28th Aug, 2020

Title:	Discussion on the remaining issues for UE measurement capabilities requirements

Source:	Xiaomi

Agenda Item:	7.14.1.3

Document for:	Discussion

- Introduction

In RAN#88-e plenary meeting, the exception sheet for CSI-RS based L3 measurement has been approved in [1]. And the following open issues are aimed to be concluded in this meeting:

- CSI-RS configuration applicability

- Scope of requirement

- UE capability

- UE measurement capability requirement

- Cell identification requirement

- Scheduling restriction

- The collision case between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell

- CSSF for CSI-RS based measurement within measurement gap and outside of measurement gap.

- Synchronization assumption for CSI-RS measurement

In this contribution, we would like to discuss the remaining issues on UE measurement capabilities requirements and provide our proposals.

- Discussion

In last meeting, RAN4 had extensive discussion on UE measurement capabilities. The related agreement were captured in agreed WF [2], which are copied as follows:

- CSI-RS measurement capability requirements are defined based on per frequency layer - FFS: CSI-RS and SSB for mobility configured in the same MO are counted as [2] layers - UE shall be able to measure at least - 1 SSB intra-frequency layer and [1] CSI-RS intra-frequency layer per serving cell - 7 SSB inter-frequency layers and [7] CSI-RS inter-frequency layers - [8] NR inter-frequency layers including SSB and CSI-RS in total, - and 13 NR inter-frequency and inter-RAT layers in total - During each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ and CSI-SINR measurements for at least: - 8 identified cells for FR1, and 6 identified cells for FR2 , for each intra-frequency layer - 4 identified cells for FR1, and 4 identified cells for FR2, for each inter-frequency layer - The number of CSI-RS resource/beams to be monitored for UE shall be at least - [14/32] CSI-RS resources for intra frequency measurements in FR1, [24/32] CSI-RS resources for intra frequency measurements in FR2, - [7] CSI-RS resources for inter frequency measurements in FR1, [14] CSI-RS resources for inter frequency measurements in FR2. - FFS the values in [] and FFS the relation to maxNumberCSI-RS-RRM-RS-SINR limitation - Per FR2 band, intra-frequency CSI-RS measurement for neighbour cells is only required for PCC/PSCC or one of SCell if there is no PCC/PSCC on that band. UE shall also be capable of at least 2 SSBs and 2 CSI-RS resources on serving cell per CC in the same band.

In TS38.331, the SSB based intra/inter-frequency requirement and CSI-RS based intra/inter-frequency requirement are configured in IE MeasObjectNR.

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

According to the configuration, the SSB based measurement for mobility and CSI-RS based measurement for mobility are configured independently, and the SSB and CSI-RS can be configured in different time and frequency domain. In addition, for CSI-RS based measurement, the separate computation and memory resources are assumed from UE implementation point of view, and UE may need additional efforts for CSI-RS based measurement e.g. sampling, buffering, processing and filtering. Hence, the SSB and CSI-RS should be counted as 2 frequency layers.

Proposal 1: CSI-RS and SSB for mobility configured in the same MO are counted as 2 layers.

Another open issue is the number of CSI-RS resource/beam to be monitored per frequency layer. In TS38.214, the maximum number of CSI-RS resource for each UE is set to be 96 per frequency layer when associated SSB is configured. According to the discussion in previous meeting, a larger number of CSI-RS resource than SSB is preferred from network implementation point of view. Thus, we propose 32 CSI-RS resources per frequency layer for both FR1 and FR2 are defined for intra-frequency, and for inter-frequency layer, 24 CSI-RS resources per frequency layer for both FR1 and FR2 are defined.

Proposal 2: For intra-frequency layer, UE shall monitor at least 32 CSI-RS resources per frequency layer for both FR1 and FR2.

Proposal 3: For inter-frequency layer, UE shall monitor at least 24 CSI-RS resources per frequency layer for both FR1 and FR2.

- Conclusion

In this contribution, we discuss the remaining issues on cell identification requirements and provide our proposals as follows:

Proposal 1: CSI-RS and SSB for mobility configured in the same MO are counted as 2 layers.

Proposal 2: For intra-frequency layer, UE shall monitor at least 32 CSI-RS resources per frequency layer for both FR1 and FR2.

Proposal 3: For inter-frequency layer, UE shall monitor at least 24 CSI-RS resources per frequency layer for both FR1 and FR2.

- Reference

[1]	RP-201340	Rel-16 Work Item Exception for CSI-RS based L3 measurement, CATT

[2]	R4-2009009	WF on CSI-RS based L3 measurement capability and requirements, OPPO