3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2009826

Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.886 | CR | 0001 | rev | - | Current version: | 16.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | REFSENS requirements for NR V2X band n38 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2020-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The noise figure of 13dB instead of 9dB should be used for n38 REFSENS requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 4dB to each REFSENS value for n38 in Table 9.1.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The REFSENS requirements for n38 would be much tighter. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.1	NR V2X UE Rx requirements in FR1

When a NR V2X UE is TDM operating between NR SL and LTE SL at n47, the NR V2X UE need to satisfy the individual V2X Rx requirements of each RAT.

9.1.1	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements.

-	IM: 2.5 dB is assumed.

-	Target SNR: [-1] dB

-	Diversity gain: 3dB

The REFSENS requirements for NR V2X Bands (PC5) are in Table 9.1.1-1.

Table 9.1.1-1: Reference sensitivity for V2X Band (PC5)

| NR Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS kHz | 10MHz (dBm) | 20MHz (dBm) | 30MHz (dBm) | 40MHz (dBm) | Duplex Mode |
| n38 | 15 | [-92.8] | [-89.8] |  | [-86.6] | TDD |
|  | 30 | [-93.1] | [-90.0] |  | [-86.7] |  |
|  | 60 | [-93.5] | [-90.2] |  | [-86.9] |  |
| n47 | 15 | [-92.8] | [-89.7] | [-87.9] | [-86.6] | HD |
|  | 30 | [-93.1] | [-89.9] | [-88.0] | [-86.7] |  |
|  | 60 | [-93.5] | [-90.1] | [-88.1] | [-86.9] |  |

Table 9.1.1-2: Sidelink TX configuration for reference sensitivity of V2X Bands (PC5)

| NR operating Band / SCS/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X Band | SCS (kHz) | 10 MHz (dBm) | 20 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | Duplex Mode |
| n38 | 15 | 52 | 106 |  | 216 | TDD |
|  | 30 | 24 | 51 |  | 106 |  |
|  | 60 | 11 | 24 |  | 51 |  |
| n47 | 15 | 52 | 106 | 160 | 216 | HD |
|  | 30 | 24 | 51 | 78 | 106 |  |
|  | 60 | 11 | 24 | 38 | 51 |  |

<End of Change 1>