3GPP TSG-RAN WG4 Meeting # 96-e 															R4-2009827

Electronic Meeting, 17-28 Aug., 2020

Source: 	CATT

Title: 	On BS impact of NR V2X

Agenda item:	7.3.2

Document for:	Approval

- Introduction

The CR on the introduction of frequency bands for NR V2X was assigned with consensus in the email discussion before RAN4#94e meeting [1]. The BS impact of NR V2X was discussed in the previous meetings but without any progress [2] [3] [4]. This contribution will further provide analysis on the necessity of the CR introduction.

- Discussion

As per the WID on NR V2X [5] approved in RAN#87e, the impact on BS specification (TS 38.104) has been added to introduce NR V2X bands. So it is agreed that NR V2X has the BS impact from the perspective of RAN plenary.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| … | … | … | … |
| 38.101-1 | Introduction of NR V2X solutions | RAN#88 | Core part |
| 38.101-2 | Introduction of NR V2X solutions | RAN#88 | Core part |
| 38.101-3 | Introduction of NR V2X solutions | RAN#88 | Core part |
| 38.101-4 | Introduction of NR V2X solutions | RAN#90 | Performance part |
| 38.104 | Introduction of NR V2X bands | RAN#88 | Core part |

In general, the frequency bands defined in BS specifications are commonly aligned with those in UE specifications from the perspective of band indicator. In Rel-14, Band 47 is specified in the section of operating bands in both TS 36.101 and TS 36.104. Given the protection of V2X UE Rx in sidelink frequency band, the sidelink frequency bands are considered BS-related instead of only UE-related. When it comes to NR V2X, the situation is unchanged and the same methodology as LTE V2X should be applied. It is also essential to introduce the sidelink frequency bands for NR V2X into BS specifications.

Proposal 1: To introduce the sidelink frequency bands for NR V2X in TS 38.104.

In LTE V2X, Band 47 is defined for V2X sidelink operation including both UE Tx and UE Rx. Whether or not to specify co-existence spurious emission requirement in BS specifications for the protection of UE Rx in Band 47 was discussed in LTE V2X. The conclusion was to specify such requirements in TS 36.104 and to approve the corresponding CR for TS 36.104 [6].

The spurious emission of the UEs operating at other bands (usually -30dBm/MHz) have the interference with V2X UE Rx. However, the spurious emission of UE is different from that of BS. UE transmits signal discontinuously with a low probability, and the distance from victim V2X UE is rather random. To conclude, the probability statistics of the interference on NR V2X can meet the coexistence requirements with -30dBm/MHz spurious emission.

On the other hand, BS is always transmitting signal with configured transmission power. If no limits specified on BS transmission, Rx of the UEs operating in the other bands, e.g. V2X UE, will be significantly impacted even though BS is far away from some UEs. To solve this kind of coexistence problem, BS spurious emission basic limits for coexistence are specified for both LTE and NR. To meet the coexistence requirements, -52dBm-MCL should be less than the REFSENS level of UE minus ∆, so the nearby UEs will not be impacted by BS.

NR V2X has the same situation as LTE V2X and thereby needs to specify co-existence spurious emission requirement to protect UE Rx in band n47. In current TS 38.104, the co-existence spurious emission requirement is only specified to protect UE Rx in Band 47 for LTE V2X without band n47 for NR V2X. The corresponding CR including the protection of V2X UE Rx in band n47 is thus derived in our companion paper [7].

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| … | … | … | … | … |
| --- | --- | --- | --- | --- |
| E-UTRA Band 46 | 5150 – 5925 MHz | -52 dBm | 1 MHz |  |
| E-UTRA Band 47 or NR Band n47 | 5855 – 5925 MHz | -52 dBm | 1 MHz |  |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -52 dBm | 1 MHz | This is not applicable to BS operating in Band n48, n77 or n78. |
| E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n74, n75, n76, n91, n92, n93 or n94. |
| E-UTRA Band 51 or NR Band n51 | 1427 – 1432 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n50, n51, n75, n76, n91, n92, n93 or n94. |
| … | … | … | … | … |

Proposal 2: To specify co-existence spurious emission requirement in TS 38.014 for the protection of UE Rx in band n47.

- Conclusion

This contribution provides our views and analysis on the introduction of frequency band and channel arrangement for NR V2X in 38.104. The following proposals are concluded as follows:

Proposal 1: To introduce the frequency bands for NR V2X in TS 38.104.

Proposal 2: To specify co-existence spurious emission requirement in TS 38.104 for the protection of UE Rx in band n47.

- References

- R4-2001215, Summary on E-mail discussion for NR V2X, LG Electronics France, RAN4#94e

- R4-2001882, Email discussion summary for RAN4#94e_#13_5G_V2X_NRSL_SysParameters, Moderator (vivo), RAN4#94e

- R4-2005692, Email discussion summary for [94e Bis][9] 5G_V2X_NRSL_SysParameters, Moderator (vivo), RAN4#94-e-bis

- R4-2008943, Email discussion summary for [95e][113] 5G_V2X_NRSL_SysParameters, Moderator (vivo), RAN4#95-e

- RP-200129, Revised WID on 5G V2X with NR sidelink, LG Electronics, RAN#87e

- R4-1704054, CR on BS for protection of V2X UE in TS 36.104, Huawei, RAN4#82bis

- R4-20xxxxx, CR for TS38.104, Introduce BS impact of NR V2X, CATT, vivo, RAN4#96-e