3GPP TSG-RAN WG4 Meeting #96-e                 	R4-2009897

Electronic Meeting, 17th – 28th Aug., 2020

Agenda item:		7.5.3.2.2

Source:	MediaTek Inc.

Title:	Discussion on dormancy SCell

Document for:		Discussion

# 1	Introduction

In last meeting [1], companies reached some agreements on UE requirements for transitions between ‘dormancy’ and ‘non-dormancy’ BWP. The remaining issues are:

At RAN4#95e, the following topics were discussed: - Sub-topic 2-1: Switching of single SCell between dormancy and non-dormancy, triggering inside active time - Issue 2-1-1: Switching delay non-dormancy to dormancy, general case w.r.t. parameter change - Issue 2-1-2: Switching delay non-dormancy to dormancy, optimized w.r.t. parameter change - Issue 2-1-3: Switching delay dormancy to non-dormancy, general case w.r.t. parameter change - Issue 2-1-4: Switching delay dormancy to non-dormancy, optimized w.r.t. parameter change - Issue 2-1-5: Interruption at switching between dormancy and non-dormancy - Sub-topic 2-2: Switching of single SCell between dormancy and non-dormancy, triggering outside active time - Issue 2-2-1: Switching delay between dormancy and non-dormancy - Issue 2-2-2: Interruption at switching between dormancy and non-dormancy - Sub-topic 2-4: CSI and RRM measurements during dormancy - Issue 2-4-1: Measurement requirements - Issue 2-4-2: Interruptions Way Forward - 2-1-1: Switching delay non-dormancy to dormancy, general case w.r.t. parameter change - Value of X. Numerology of Z. - 2-1-2: Switching delay non-dormancy to dormancy, optimized w.r.t. parameter change - Whether to have separate requirements for certain parameter changes. - 2-1-5: Interruption at switching between dormancy and non-dormancy - Whether toggling RF on/off shall be allowed. Whether SCS change results in cross-FR interruption. - 2-2-1: Switching delay between dormancy and non-dormancy - 2-2-2: Interruption at switching between dormancy and non-dormancy - 2-4-2: Interruptions for CSI and RRM measurements during dormancy - Whether to have ACK/NACK rate, or ACK/NACK rate plus interruption window.

Besides, RAN1 also sent an LS [2] and listed some questions for RAN4 to reply.

In this paper, we will discuss the related open issues of UE requirements for dormancy SCell.

# 2	Discussion

In RAN4 #95-e meeting, companies agreed that for inside active time DCI-based dormancy switching, the switching delay should follow requirements below:

Agreement: - For DCI-based triggering with DCI received in any of the first X OFDM symbols of a slot, and for timer-based triggering, the switching delay for transition from non-dormancy to dormancy is given by Table 8.6.2-1. - For DCI-based triggering with DCI received after the first X OFDM symbols of a slot, if applicable, the switching delay for transition from non-dormancy to dormancy is given by Table 8.6.2-1 plus Z additional slot(s). - For value of X: - Option 1a: X = 3 symbols - Option 1b: X = 7 symbols - For value of Z: - Option 2a: Z = 1 slot in numerology for the spCell in which the triggering DCI was received - Option 2b: Z = 1 slot in the numerology for the SCell for which the transition is triggered - In case SCS differs between spCell and SCell, the switching delay, except for Z slot(s), associated with the smaller SCS applies.

Here, the value of X and numerology of Z are not yet determined. Considering that the DCI is received on the spCell, Z=1 slot should be counted based on this spCell’s SCS. As for the value of X, we can follow the majority view to set X=3 symbols, but the pre-condition on this X=3 symbols was missed in RAN4’s discussion, i.e., whether UE can support PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot is a UE capability. RAN4 should only specify the corresponding requirement for UE who supports

- pdcch-MonitoringAnyOccasionsWithSpanGap, or

- pdcch-MonitoringAnyOccasions.

So we propose

Proposal 1: For UE supporting pdcch-MonitoringAnyOccasionsWithSpanGap or pdcch-MonitoringAnyOccasions, switching delay is extended by additional one slot (based on SpCell SCS) if DCI is received after first 3 symbols in a slot; otherwise, DCI is expected to be received in first 3 OFDM symbols of a slot and the switching delay between non-dormancy and dormancy shall follow the Rel-15 BWP switching delay specified in Table 8.6.2-1

Regarding to the interruption at switching between dormancy and non-dormancy BWP, we agree with other companies’ view that toggling RF on/off should be allowed, so that UE can save more power in dormant BWP. Furthermore, once UE conducts the RF retuning or baseband resource re-allocation on the SCell, it might also interrupt the transmission of ACK/NACK feedback on the spCell. RAN4 also needs to inform RAN1 that restriction, so we suggest that

Proposal 2: Interruption length in Table 8.2.2.2.5-1 always applies when UE switches between non-dormancy BWP and dormancy BWP. Before and during the interruption, UE is not expected to transmit ACK/NACK feedback for non-scheduling DCI as well as receive PDSCH and transmit PUSCH scheduled by dormancy indication DCI

For issue 2-4-2, we agree that it is beneficial for Network scheduling if interruption window can also be specified for CSI-RS and RRM measurement during SCell dormancy. So option 2 is also acceptable

Proposal 3: RAN4 to specify both ACK/NACK missed rate and interruption window for CSI and RRM measurements during SCell dormancy

# 5	Summary

In this contribution, we have the following proposals:

Proposal 1: For UE supporting pdcch-MonitoringAnyOccasionsWithSpanGap or pdcch-MonitoringAnyOccasions, switching delay is extended by additional one slot (based on SpCell SCS) if DCI is received after first 3 symbols in a slot; otherwise, DCI is expected to be received in first 3 OFDM symbols of a slot and the switching delay between non-dormancy and dormancy shall follow the Rel-15 BWP switching delay specified in Table 8.6.2-1

Proposal 2: Interruption length in Table 8.2.2.2.5-1 always applies when UE switches between non-dormancy BWP and dormancy BWP. Before and during the interruption, UE is not expected to transmit ACK/NACK feedback for non-scheduling DCI as well as receive PDSCH and transmit PUSCH scheduled by dormancy indication DCI

Proposal 3: RAN4 to specify both ACK/NACK missed rate and interruption window for CSI and RRM measurements during SCell dormancy

# Reference

- R4-2008607, Way Forward on SCell Dormancy, Ericsson, Jun., 2020.

- R1-2005081, LS on SCell Dormancy, Ericsson, Jun., 2020.