3GPP RAN4#96-e	R4-2009988

Online Meeting, August 17th – August 28th, 2020

Title: 	General requirements in IAB networks

Source: 	Qualcomm Incorporated

Agenda item:	7.4.1.1

Document for:	Discussion

# Introduction

RAN4 has been discussing the requirements of IAB networks during several meetings [1]. There are still a few minor TBD items left in the spec: including the regional requirements and requirements for contiguous and non-contiguous spectrum in IAB networks. We provide a TP for those topics in this contribution.

# Discussion

RAN4 has already defined requirements for contiguous and non-contiguous spectrum for base stations [3]. IAB-DUs can reuse those requirements.

3GPP has also defined the applicability of requirements for BS type 1-H, 1-O and 2-O. Those same requirements can be applicable for IAB-DU type 1-H, 1-O and 2-O respectively.

Proposal 1: The requirements for contiguous and non-contiguous spectrum for IAB-DUs are same as those of base stations.

Proposal 2: The applicability of requirements for IAB-DU type 1-H, 1-O and 2-O are same as those of BS type 1-H, 1-O and 2-O respectively.

# TP for Section 4 of 38.174

-------------------------------------------------------------------  TP starts ----------------------------------------------------------

# General

## Relationship with other core specifications

The present document is a single-RAT specification for an IAB_DU and IAB-MT, covering RF characteristics and minimum performance requirements and RRM requirements for the IAB_MT. Conducted and radiated core requirements are defined for the IAB node architectures and IAB node types defined in subclause 4.3.

The applicability of each requirement is described in clause 4.6.

## Relationship between minimum requirements and test requirements

Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification [Test specification references].

The minimum requirements given in this specification make no allowance for measurement uncertainty. The test specifications [Test specification references] define test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in recommendation ITU-R M.1545 [18].

## Conducted and radiated requirement reference points

### IAB type 1-H

For IAB type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements.

Figure 4.3.2-1: Radiated and conducted reference points for IAB type 1-H

Radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Conducted characteristics are defined at individual or groups of TAB connectors at the transceiver array boundary, which is the conducted interface between the transceiver unit array and the composite antenna.

The transceiver unit array is part of the composite transceiver functionality generating modulated transmit signal structures and performing receiver combining and demodulation.

The transceiver unit array contains an implementation specific number of transmitter units and an implementation specific number of receiver units. Transmitter units and receiver units may be combined into transceiver units. The transmitter/receiver units have the ability to transmit/receive parallel independent modulated symbol streams.

The composite antenna contains a radio distribution network (RDN) and an antenna array. The RDN is a linear passive network which distributes the RF power generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array, in an implementation specific way.

How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement subclause.

### IAB type 1-O and IAB type 2-O

For IAB type 1-O and IAB type 2-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement.

Figure 4.3.3-1: Radiated reference points for IAB type 1-O and IAB type 2-O

For an IAB-DU type 1-O the transceiver unit array must contain at least 8 transmitter units and at least 8 receiver units. Transmitter units and receiver units may be combined into transceiver units. For IAB-MT the transceiver unit array must contain at least FFS transmitter units and FFS receiver units. The transmitter/receiver units have the ability to transmit/receive parallel independent modulated symbol streams.

## IAB classes

### IAB-DU classes

The requirements in this specification apply to Wide Area IAB-DU, Medium Range IAB-DU and Local Area IAB-DU unless otherwise stated. The associated deployment scenarios for each class are exactly the same for IAB-DU with and without connectors.

For IAB type 1-O and 2-O, IAB-DU classes are defined as indicated below:

-	Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with an IAB-DU to UE minimum distance along the ground equal to 35 m.

-	Medium Range IAB-DU are characterised by requirements derived from Micro Cell scenarios with an IAB-DU to UE minimum distance along the ground equal to 5 m.

-	Local Area IAB-DU are characterised by requirements derived from Pico Cell scenarios with an IAB-DU  to UE minimum distance along the ground equal to 2 m.

For IAB type 1-H, IAB-DU classes are defined as indicated below:

-	Wide Area IAB-DU are characterised by requirements derived from Macro Cell scenarios with an IAB-DU to UE minimum coupling loss equal to 70 dB.

-	Medium Range IAB-DU are characterised by requirements derived from Micro Cell scenarios with an IAB-DU to UE minimum coupling loss equals to 53 dB.

-	Local Area IAB-DU are characterised by requirements derived from Pico Cell scenarios with an IAB-DU to UE minimum coupling loss equal to 45 dB.

### IAB-MT classes

Detailed structure of the sub clause is TBD

## Regional requirements

Detailed structure of the subclause is TBD.

## Applicability of requirements

In table 4.6-1, the requirement applicability for each requirement set is defined. For each requirement, the applicable requirement clause in the specification is identified. Requirements not included in a requirement set is marked not applicable (NA).

Table 4.6-1: Requirement set applicability

| Requirement |
| --- |
|  | IAB type 1-H | IAB type 1-O | IAB type 2-O |
| Output power | 6.2 | NA | NA |
| Output power dynamics | 6.3 |  |  |
| Transmit ON/OFF power | 6.4 |  |  |
| Transmitted signal quality | 6.5 |  |  |
| Occupied bandwidth | TBD |  |  |
| ACLR | TBD |  |  |
| Operating band unwanted emissions | TBD |  |  |
| Transmitter spurious emissions | TBD |  |  |
| Transmitter intermodulation | 6.7 |  |  |
| Reference sensitivity level | 7.2 |  |  |
| Dynamic range | 7.3 |  |  |
| In-band selectivity and blocking | 7.4 |  |  |
| Out-of-band blocking | 7.5 |  |  |
| Receiver spurious emissions | 7.6 |  |  |
| Receiver intermodulation | 7.7 |  |  |
| In-channel selectivity | 7.8 |  |  |
| Performance requirements | 8 |  |  |
| Radiated transmit power | 9.2 | 9.2 | 9.2 |
| OTA Output power | NA | 9.3 | 9.3 |
| OTA output power dynamics |  | 9.4 | 9.4 |
| OTA transmit ON/OFF power |  | 9.5 | 9.5 |
| OTA transmitted signal quality |  | 9.6 | 9.6 |
| OTA occupied bandwidth |  | TBD | TBD |
| OTA ACLR |  | TBD | TBD |
| OTA out-of-band emission |  | TBD | TBD |
| OTA transmitter spurious emission |  | TBD | TBD |
| OTA transmitter intermodulation |  | 9.8 | NA |
| OTA sensitivity | 10.2 | 10.2 | NA |
| OTA reference sensitivity level | NA | 10.3 | 10.3 |
| OTA dynamic range |  | 10.4 | NA |
| OTA in-band selectivity and blocking |  | 10.5 | 10.5 |
| OTA out-of-band blocking |  | 10.6 | 10.6 |
| OTA receiver spurious emission |  | 10.7 | 10.7 |
| OTA receiver intermodulation |  | 10.8 | 10.8 |
| OTA in-channel selectivity |  | 10.9 | 10.9 |
| Radiated performance requirements |  | 11 | 11 |

## Applicability of RRM requirements in this specification

### Applicability of signalling characteristics related RRM requirements

The RRM requirements on the signalling characteristics for IAB MTs specified in section 12.3 shall apply only for the local area IAB class defined in section 4.4.

## Requirements for contiguous and non-contiguous spectrum

The BS requirements for contiguous and non-contiguous spectrum specified in 4.7 in TS 38.104 [2] apply to IAB-DUs.

-------------------------------------------------------------------  TP ends ----------------------------------------------------------

# References

- Chairman’s notes, 3GPP TSG RAN WG4 #95e.

- 3GPP TS 38.174 V0.1.0, Integrated Access and Backhaul Radio Transmission and Reception (Release 16).

- 3GPP TS 38.104