3GPP TSG-WG4 Meeting #96-e	R4-2010014

# Electronic Meeting, 17th – 28th August, 2020

Source: 	Qualcomm Incorporated

Title: 	V2X TX Diversity, TX external components, Switching time between NR SL and LTE SL

Agenda item:	7.3.3.1

Document for:	Approval

# 1.0 Introduction

In the past several meeting there has been a great deal of discussion regarding TX diversity in the main forum. These discussions are still ongoing and the exact way to define measurements from multiple antenna have not been fully resolved [1].  This paper proposes a way forward to defining TX diversity for V2X.

In large form factor implementations such as installation of V2X modules in vehicles external components may be used to move the antenna away from the UE. This leads to an ambiguity as to the reference point for the transceiver specifications. To resolve this ambiguity this paper proposes a clarification to the TS38.101-1 specification for the case when external components are used.

The switching time between NR SL and LTE SL in TDM operations has been discussed in previous RAN4 meetings [2,3] and several numbers have been proposed.  In the following document switching time between NR SL and LTE SL requirement is proposed.

# 2.1 TX diversity for V2X

There has been much discussion in the main RAN4 forum on how to specify TX diversity for NR. Also, there have been separate discussions to introduce TX diversity in the RAN4 V2X forum.

In RAN4#95-e a WF [1] was approved that attempted to resolve issues pertaining to enabling TX diversity in the main forum. Though some issues were resolved others still need to be discussed further in the upcoming RAN4 meetings. Given that these discussions are ongoing in the main forum we think that it is best to wait until these issues are resolved before introducing a TX diversity CR to the V2X forum. This would be the best course of action as decisions in the main forum may have an impact on the specification of TX diversity in V2X and premature adoption of specifications in V2X may lead to them having to be changed in order to harmonize the TX diversity specifications between the 2 forums.

Proposal 1: Wait until NR TX diversity has been finalized in the main forum before specifying it for V2X

We also think that it is logical to follow the decisions made in the main forum when specifying TX diversity for V2X. This way the specifications can be harmonized so that there is commonality between the TX diversity specifications in the main forum and those in V2X.

Proposal 2: Follow decisions made in the main forum for TX diversity when specifying TX diversity for V2X.

In the appendix we present an example CR for V2X TX diversity which may be used for introducing V2X TX diversity into the TS38.101-1 specification. This example illustrates the sections in the V2X specification that we think need to be modified to introduce TX diversity.

2.2 Clarification of location for TX measurements

Figure 1. UE with external cable and compensator for V2X application

In RAN4 the common understanding is that the UE antenna connector is located at the output of the UE as indicated in figure 1. In V2X applications due to its large form factor there may be an external vehicle antenna connector placed away from the UE. In this scenario there is potential for uncertainty as to whether the transceiver specifications are referenced to the UE antenna connector or the vehicle antenna connector.  To resolve this ambiguity wording should be inserted into TS38.101-1 in section 6.1 to clarify the reference point for the transmitter specifications as being at the UE antenna connector and to also state that any components external to the UE used to move the UE antenna connector to any other location will not be considered in these specifications. It is proposed that the following wording be added to sections 6.1 of TS38.101-1:

- 6	Transmitter characteristics

- 6.1	General

- Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s).For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed For V2X applications the transmitter characteristics are specified at UE antenna connector without any external components that may be used to relocate the antenna away from the UE. An example of a V2X UE integration block diagram is illustrated in annex G. Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

The sentences added to section 6.1 are indicated in bold text. The V2X UE integration block diagram in annex G of TS38.101-1 and associated text is given below.

Annex G of TS38.101-1 (Informative)

- Figure G.1-1 shows a V2X integration block diagram with an example set of components external to the UE such as cables and compensator devices.

Figure 2. V2X UE integration block diagram and associated text for annex G in TS38.101-1

Proposal 3: Revise wording in section 6.1 of TS38.101-1 as outlined above

Proposal 4: Add V2X UE integration block diagram in figure 2 to annex G of TS38.101-1

2.3 Switching time between NR SL and LTE SL

In the past meetings there have been numerous opinions expressed on the NR SL to LTE SL switching time [4,5]. For a 1TX/2RX RF architecture where a common PA is shared between LTE SL and NR SL the LO frequency retune time has been calculated to be 140us and the RF chain reconfiguration time to be 70us giving a total switching time of 210us. This transient time will apply for both contiguous and non-contiguous NR/LTE spectral allocations. A switching time of 210 us is also being considered for SUL and UL CA switching [4].

Many companies proposed 150us switching time instead of 210us, and we understand that some implementation can achieve faster switching time. However, allowing longer switching time can save power by turning off more components on RF front end when the other RAT is transmitting, also simplify the switching implementation by avoiding complicated schemes for faster switching, such as parallelizing reconfiguration, complex algorithms for controlling PLL settling and reprogramming. We understand that some companies are choosing a more complicated switching implementation and keep more RF components on to reduce the switching time, we are not against such implementation. However, in RRM session the following scheduling restriction was agreed in RAN4#95e:

When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in NR slot ‘n’,

- UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n’.

When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in NR slot ‘n-1’,

- UE is not expected to transmit or receive on NR V2X sidelink on the slot ‘n-1’.

When switch from NR V2X sidelink to E-UTRA V2X sidelink occurs in E-UTRA subframe ‘n’,

- UE is not expected to transmit or receive on E-UTRA V2X sidelink on the subframe ‘n’.

When switch from E-UTRA V2X sidelink to NR V2X sidelink occurs in E-UTRA subframe ‘n-1’,

- UE is not expected to transmit or receive E-UTRA on V2X sidelink on the subframe ‘n-1’.

Therefore, no matter how fast UE can switch between NR and LTE Tx, at least in one slot or subframe UE is not expected to receive/transmit any data. Since both 210us and 150us are shorter than slot length in FR1, the corresponding system performance is exactly the same for both switching time requirement, based on the RRM requirement.

Unless companies can show that 150us can enhance system performance, otherwise we don’t agree to a requirement which may lead to sacrifice power and implementation complexity while not gaining any benefit on performance. The only corner case that 150us and 210us can be distinguished is when UE is configured with 11 symbols in 15kHz SCS, or 8 or 9 symbols in 30kHz SCS, and UE can utilize the empty symbols to switch Tx across RATs. However, supporting less than 14 symbols reception and transmission is an optional feature for dedicated bands for sidelink. Verifying an essential function of cross-RAT Tx switching by an optional feature is not a feasible way to guarantee system performance, therefore instead of specifying this requirement in RF specification, we propose to only specify this in RRM specification, as already agreed requirement in 38.133 clause 12.9.1.

Proposal 5: Do not specify switching time RF requirement, use RRM requirement in 38.113 clause 12.9.1 to verify the LTE-NR Tx switching requirement.

However, since this issue has been discussed for many meetings, in case companies can not agree to Proposal 1, we would like to propose the following compromised proposal as package covering the switching time and how UE satisfies the requirements:

- Switching time requirement in RF spec (38.101) is 150us

- If UE supports this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement can be verified by configuring smaller number of consecutive symbols in a slot, and UE has to satisfy the 150us switching time requirement.

- If UE doesn’t support this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement is considered as satisfied when UE satisfied the corresponding scheduling restriction requirement in 38.133 clause 12.9.1.

Proposal 6: LTE-NR Tx Switching time requirement is specified as a package in the following:

- Switching time requirement in RF spec (38.101) is 150us

- If UE supports this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement can be verified by configuring smaller number of consecutive symbols in a slot, and UE has to satisfy the 150us switching time requirement.

- If UE doesn’t support this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement is considered as satisfied when UE satisfied the corresponding scheduling restriction requirement in 38.133 clause 12.9.1.

Switching position is discussed in the previous meeting, we provide our view on this issue below. Our assumption is that the boundary between the LTE subframe and the NR slot is time aligned so if the switching time is greater than the guard period an interruption time of either 1 NR slot or 1 LTE subframe must be generated. Below the various switching scenarios are explained in detail.

Consider the SL UE is transmitting with current RAT at slot/subframe n, and want to switch to the other RAT at slot/subframe n+1. Let switching time be S us, and guard symbol in current RAT g. According to the WF agreed in RAN4#94ebis, S > g and S < NR slot length when we consider FR1. We consider NR slot has fully allocated symbols. The following scenarios are possible:

- Current RAT is LTE, switch to NR

(1) 	Switch happens at subframe n and S us before n+1 slot boundary: interruption is 1 subframe on LTE

(2) Switch happens at subframe n and less than S us but more than g us before n+1 slot boundary: interruption is 1 subframe on LTE and 1 slot on NR

(3) Switch happens at subframe n and less than g us before n+1 slot boundary or later: interruption is 1 slot on NR

Similarly, we can derive:

- Current RAT is NR, switch to LTE

(1) 	Switch happens at slot n and S us before n+1 subframe boundary: interruption is 1 slot on NR

(2) Switch happens at slot n and less than S us but more than g us before n+1 subframe boundary: interruption is

1 subframe on LTE and 1 slot on NR

(3) Switch happens at slot n and less than g us before n+1 subframe boundary or later: interruption is 1 slot on LTE

From above analysis, we can conclude that the interruption is at least one slot on NR or one subframe on LTE, and at most one slot on NR AND one subframe on LTE. In order to minimize interruption, UE should choose either position (1) or (3) in above analysis. If either (1) or (3) is selected, the interruption is one slot on NR or one subframe on LTE, under the assumptions of S > g and S < NR slot length.

Observation 1: Regardless of switch position being chosen, the interruption is at least one slot on NR or one subframe on LTE, and at most one slot on NR AND one subframe on LTE.

Observation 2: For switch happens at (1) slot/subframe n and S us before n+1 subframe/slot boundary, or (2) slot n and less than g us before n+1 subframe boundary or later, one slot on NR or one subframe LTE interruption is achievable.

# Conclusion

This paper presents proposals on how best to defined TX diversity for V2X which are given below.

Proposal 1: Wait until NR TX diversity has been finalized in the main forum before specifying it for V2X

Proposal 2: Follow decisions made in the main forum for TX diversity when specifying TX diversity for V2X.

This paper also makes proposals on how best to resolve the TX specification reference point when the antenna is moved away from the UE using components external to the UE.

Proposal 3: Revise wording in section 6.1 TS38.101-1 as outlined above

Proposal 4: Add V2X UE integration block diagram in figure 2 to annex G of TS38.101-1

This contribution also gives the transient time for switching between NR SL to LTE SL for both contiguous and non-contiguous spectral allocations for operation in band n47.

Proposal 5: Do not specify switching time RF requirement, use RRM requirement in 38.113 clause 12.9.1 to verify the LTE-NR Tx switching requirement.

Proposal 6: LTE-NR Tx Switching time requirement is specified as a package in the following:

- Switching time requirement in RF spec (38.101) is 150us

- If UE supports this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement can be verified by configuring smaller number of consecutive symbols in a slot, and UE has to satisfy the 150us switching time requirement.

- If UE doesn’t support this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement is considered as satisfied when UE satisfied the corresponding scheduling restriction requirement in 38.133 clause 12.9.1.

Observation 1: Regardless of switch position being chosen, the interruption is at least one slot on NR or one subframe on LTE, and at most one slot on NR AND one subframe on LTE.

Observation 2: For switch happens at (1) slot/subframe n and S us before n+1 subframe/slot boundary, or (2) slot n and less than g us before n+1 subframe boundary or later, one slot on NR or one subframe LTE interruption is achievable.

## Appendix 1:	Sample V2X TX diversity CR

## <Start of Changes>

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

The applicability of transmitter requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

## <Next Change>

## 6.2E	Transmitter power for V2X

### 6.2E.1	UE maximum output power for V2X

For power class 2 NR V2X UE with two transmit antenna connectors, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2E.1-1. The requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2. For NR V2X UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

For V2X UE supporting Transmit Diversity, if the UE transmits on two connectors at the same time, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2E.1-1. The maximum output power is defined as the sum of the maximum output power from both transmit antenna connectors. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1-1: NR V2X UE Power Class for SL-MIMO and Transmit Diversity

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n38 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n47 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |

If the UE transmits on one antenna connector at a time, the requirements in Table 6.2.1-1 shall apply to the active antenna connector.

### 6.2E.2	UE maximum output power reduction for V2X

For NR V2X UE with two transmit antenna connectors, the allowed Maximum Power Reduction (MPR) values specified in Table 6.2.2-1 for PC3 in Table 6.2.2-2 for PC2 shall apply to the maximum output power specified in Table 6.2E.1-1. The requirements shall be met with SL MIMO configurations defined in Table 6.2D.1-2. For UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2E.4 apply.

### 6.2E.3	UE additional maximum output power reduction for V2X

For UE with two transmit antenna connectors, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2E.1-1. The requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2. For UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2E.4 apply.

## <Next Change>

### 6.2E.4	Configured output power for V2X

For NR V2X UE supporting SL MIMO, the transmitted power is configured per each UE.

For NR V2X UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2E.4-1. The requirements shall be met with SL MIMO configurations specified in Table 6.2D.1-2.

For V2X UE supporting Transmit Diversity, the transmitted power is configured per each UE.

If the UE transmits on two antenna connectors at the same time, the tolerance is specified in Table 6.2E.4-1 V2X UE.

Table 6.2E.4-1: PCMAX,c tolerance schemes MIMO and Transmit Diversity

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

## <Next Change>

### 6.3E.1	Minimum output power for V2X

For NR V2X UE with two transmit antenna connectors, the minimum output power is defined as the sum of the mean power from both antenna connectors in one sub-frame (1 ms). The minimum output power shall not exceed the values specified for single carrier.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the minimum output power is defined as the sum of the mean power from both transmit connectors in one sub-frame (1ms). The minimum output power shall not exceed the values specified for single carrier.

If the UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

### 6.3E.2	Transmit OFF power for V2X

The transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power at each transmit antenna connector shall not exceed the values specified for single carrier.

## <Next Change>

### 6.3E.3	Transmit ON/OFF time mask for V2X

For NR V2X UE supporting SL MIMO, the ON/OFF time mask requirements apply at each transmit antenna connector.

For UE with two transmit antenna connectors, the general ON/OFF time mask requirements specified in clause 6.3E.3 apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

For V2X UE supporting Transmit Diversity, the ON/OFF time mask requirements apply at each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the general ON/OFF time mask requirements apply to the active antenna connector.

## <Next Change>

### 6.3E.4	Power control for V2X

#### 6.3E.4.3	Power control for V2X con-current operation in licensed band

For NR V2X UE supporting SL MIMO, the power control tolerance for single carrier shall apply to the sum of output power from both transmit antenna connectors.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the power control tolerance for single carrier shall apply to the sum of output power from both transmit antenna connectors.

If the UE transmits on one -antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

## <Next Change>

## 6.4E	Transmit signal quality for V2X

### 6.4E.1	Frequency error for V2X

For NR V2X UE supporting SL MIMO, the UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ±0.1 PPM observed over a period of 0.5 ms in case of using GNSS synchronization source. The same requirements applied over a period of 0.5 ms compared to the relative frequency in case of using the NR gNode B or V2X UE sidelink synchronization signals.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) in case of using GNSS synchronization source. The same requirements applied over a period of one time slot (0.5 ms) compared to the relative frequency in case of using the NR Node B or V2X UE sidelink synchronization signals.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

### 6.4E.2	Transmit modulation quality for V2X

For NR V2X UE supporting SL MIMO, the transmit modulation quality requirements for single carrier shall apply to  each transmit antenna connector.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna-connectors at the same time, the transmit modulation quality requirements for single carrier shall apply to each transmit antenna connector.

If V2X UE transmits on one-antenna connector at a time, the requirements specified for single carrier apply to the active antenna connector.

#### 6.4E.2.1	Error Vector Magnitude for V2X

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna-connectors at the same time, the EVM is given by

EVM=P1*EVM12+ P2*EVM22P1 + P2

Where P1, P2, EVM1 and EVM2 are the individual output powers and EVMs at each antenna connector

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

## <Next Change>

## 6.5E	Output RF spectrum emissions for V2X

### 6.5E.1	Occupied bandwidth for V2X

For NR V2X UE with two transmit antenna connectors, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in table 6.5.1-1. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the requirements for occupied bandwidth is specified at each transmit antenna connector and the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified for single carrier.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

### 6.5E.2	Out of band emission for V2X

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

Table 6.5G.2-1: ACLR requirements for power class 2 V2X Communication

|  | Channel bandwidth / ACLR / Measurement bandwidth |  |
| --- | --- | --- |
|  | 10 MHz | 20 MHz |
| ACLR | 31 dB | 31 dB |
| NR channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +10 / -10 | +20 / -20 |

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the emissions requirements specified for a single carrier apply to the sum of the emissions from the two transmit antenna connectors.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

#### 6.5E.2.4.1 ACLR for TX diversity for V2X

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna-connectors at the same time, the ACLR is given by

ACLRUE=PADI,TX1+PADI,TX2 POWN,TX1+ POWN,TX2

ADJ, TX1 = power of the adjacent channel on TX port 1

POWN, TX1 = power of own channel on TX port 1

PADJ, TX2 = power of the adjacent channel on TX port 2

POWN, TX2 = power of own channel on TX port 2

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

### 6.5E.3	Spurious emission for V2X

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to the sum of the emissions from the two antenna connectors. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

## <Next Change>

### 6.5E.4	Transmit intermodulation for V2X

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the requirements specified for single carrier shall apply for the sum of both transmit antenna connectors. If the UE transmits on one antenna connector, the requirements specified for single carrier shall apply to the active antenna connector.

## <End of Changes>

## <Next Change>

## 6.8E	Time alignment error

For V2X UE(s) with two transmit antenna connectors in SL MIMO or Transmit Diversity scheme, this requirement applies to slot timing differences between transmissions on two transmit antenna connectors.The Time Alignment Error (TAE) shall not exceed 260 ns.

## <End of Changes>

# References

- R4-2008465, “WF on Enabling Transparent TxD in Rel-16”, RAN4#95-e, May-June 2020

- R4-1912514, “On switching time between LTE SL and NR SL”, RAN4#92Bis, Chongqing, China, October 2019

- R4-1913954,” TP on additional on/OFF Switching Time Mask for TDM operation between LTE SL and NR SL at n47”, LG Electronics , RAN4#93, Reno, USA, November 2019

- R4-2002815, “WF on RF requirement for Tx switching between two uplink carriers”, RAN#94-e, Feb-Mar 2020

- R4-2004469 “Switching time between NR SL and LTE SL”, RAN#94-e-bis, 20th-30th April 2020.