3GPP TSG-RAN4 Meeting #95e	R4-2010054

# Online, August 17-28, 2020

Agenda item:	7.14.1.5

Source:	Apple

Title:   On other remaining issues of CSI-RS based L3 measurement

Document for:	Approval

# Introduction

In this contribution, the following two issues are discussed

- FFS on synchronization assumption for CSI-RS measurement

- FFS on the collision case between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell

# Synchronization assumption for CSI-RS measurement

It has been agreed the associatedSSB has to be configured for CSI-RS resource used for L3 measurement. CSI-RS and the corresponding associatedSSB should share identical timing information. To avoid unnecessary complication, it is reasonable to assume that the associated SSB has to be configured in MO and UE should be provided with the measurement opportunities for the associated SSB.

Meanwhile, we think it is unrealistic to assume single FFT to measure the CSI-RS from different cells. Even with an assumption of 100% asynchronization at NW side, the propagation delay difference can be easily exceeding CP length.

Proposal 1: The timing of CSI-RS resource should be assumed the same as the associatedSSB.

Proposal 2: No single FFT is assumed to measure CSI-RS resources from different cells.

# Collision case between L1 measurement of serving cell and CSI-RS L3 measurement of neighbor cell

Table 1: Measurement restriction and measurement resource sharing

| Measurement restriction or measurement sharing | SSB based BM/RLM | CSI-RS based BM/RLM | SSB based L3 measurement | CSI-RS based L3 measurement |
| --- | --- | --- | --- | --- |
| SSB based BM/RLM | NA | Measurement restriction (e.g. section 8.5.2.3 in TS38.133) | Measurement sharing (sharing factor P in TS38.133) | Not defined yet |
| CSI-RS based BM/RLM | Measurement restriction (e.g. section 8.5.2.3 in TS38.133) | Measurement restriction (e.g. section 8.5.3.3 in TS38.133) | Measurement sharing (sharing factor P in TS38.133) | Not defined yet |
| SSB based L3 measurement | Measurement sharing (sharing factor P in TS38.133) | Measurement sharing (sharing factor P in TS38.133) | NA | Not defined yet |
| CSI-RS based L3 measurement | Not defined yet | Not defined yet | Not defined yet | Not defined yet |

With CSI-RS based L3 measurement introduced, the related measurement opportunity sharing with existing SSB based L1/L3 measurement and CSI-RS based L1 measurement should be specified. In Table 1, the missing parts are identified. Since this is the last meeting to complete this WI, it is proposed to prioritize the measurement opportunity sharing between CSI-RS and SSB based L3 measurement and leave the rest issues to R17.

Proposal 3: Strive to complete measurement opportunity sharing between CSI-RS and SSB based L3 measurement in R16

Proposal 4: Postpone the other open issues in table 1 to Rel-17

# Conclusion

In this contribution, the following two issues are discussed

- FFS on synchronization assumption for CSI-RS measurement

- FFS on the collision case between L1 measurement of serving cell and CSI-RS L3 measurement of neighbor cell

It is proposed that

Proposal 1: The timing of CSI-RS resource should be assumed the same as the associatedSSB.

Proposal 2: No single FFT is assumed to measure CSI-RS resources from different cells.

Proposal 3: Strive to complete measurement opportunity sharing between CSI-RS and SSB based L3 measurement in R16

Proposal 4: Postpone the other open issues in table 1 to Rel-17