3GPP TSG-RAN WG4 Meeting #96-e                                        	R4-2010093

Electronic meeting, 17 – 28 August, 2020

Source: 	LG Electronics

Title: 	TP on the general part of TR 36.717-03-02

Agenda item:	14.5.3

Document for:	Approval

- Introduction

The general part of TR 36.717-03-02 (LTE-A inter-band CA for x bands (x=3, 4, 5) DL with 2 bands UL) in Rel-17 is provided in this contribution. The general part includes an example of UE general RF architecture, general treatment of ∆TIB and ∆RIB, and general TX/RX requirements.

### ****************** Start of the TP in subclause 5 of TR36.717-03-02 ************************

5	LTE-A inter-band CA for x bands DL with 2 bands UL: General part

5.1	UE RF part

5.1.1	UE general RF architecture

The UE RF architecture of the general part in TR 36.716-03-02 (Rel-16) can be reused to define Maximum Sensitivity Degradation (MSD) for LTE-A inter-band CA for x bands (x = 3, 4, 5) DL with 2 bands in Rel-17. This UE general RF architecture is for TDD-FDD Carrier Aggregation (CA) band combinations in Rel-17 and it only applies to LTE. Non-Standalone (NSA) DC/CA operation isn’t considered in this technical report.

Figure 5.1.1- 1 Example of UE RF architecture using cascaded diplexer

5.1.2	General treatment of ∆TIB and ∆RIB values

Additional insertion loss from LTE-A inter-band CA for x bands DL / 1 UL can be reused when new RF components are not necessary to be introduced. If there will be a need for introducing the new RF components, then more detail information shall be provided for a certain band combination.

5.1.3	Fallback CA mode and mandatory support of all paired 2 UL CA configurations

Based on the agreed Way forward (R4-168853) in Rel-14, fallback CA mode has been defined by RAN4.

5.1.4	General TX/RX requirements

5.1.4.1 Transmitter requirements

For dual uplink inter-band CA operation, transmitter requirements have been analyzed in TR36.860 and UEs supporting LTE-A inter-band CA for x bands (x= 3, 4, 5) DL with 2 bands UL can apply the common Tx requirements in Table 5.1.4.1-1.

Table 5.1.4.1-1: UE RF TX requirements for dual uplink inter-band CA

| Section / Clause in TS36.101 | Description |
| --- | --- |
| 5.6A.1 | Channel bandwidths per operating band for CA |
| 6.2.2A | UE maximum output power for CA |
| 6.2.5A | Configured transmitted Power for CA |
| 6.3.2A | UE Minimum output power for CA |
| 6.3.3A | UE Transmit OFF power for CA |
| 6.3.4A | ON/OFF time mask for CA |
| 6.3.5A | Power control for CA |
| 6.5.1A | Frequency error for CA |
| 6.5.2A | Transmit modulation quality for CA |
| 6.6.1A | Occupied bandwidth for CA |
| 6.6.2.1A | Spectrum emission mask for CA |
| 6.6.2.3 | Adjacent Channel Leakage Ratio |
| 6.6.3.1A | Spurious Emission for CA |
| 6.6.3.2A | Spurious emission band UE co-existence for CA |
| 6.7.1A | Transmit intermodulation for CA |

5.1.4.2 Receiver requirements

Like the transmitter requirements in the section 5.1.4.1, receiver requirements have been analysed in TR36.860 for dual uplink inter-band CA operation and additional requirements of UE RF receiver for LTE-A inter-band CA for x bands (x = 3, 4, 5) DL with 2 bands UL can be found in Table 5.1.4.2-1. For some band combinations that are related to x bands (x = 3, 4, 5) DL with 2 bands UL without self-interference problem, the same requirements of x bands DL/ 1 bands UL CA can be reused. The requirements (out-of-band blocking and spurious response) of 2 bands DL / 2 bands UL can be covered by applying the test method of one uplink (i.e. single carrier LTE and 2 bands DL / 1 band UL CA. Therefore, there will be no required to test the requirements (out-of-band blocking and spurious response) for x bands DL with 2 bands UL. Only REFSENS test will be required when all x bands DL and 2 bands UL are activated at the same time. Lastly, a new MSD value shall be defined for a band combination which has self-interference problem when two uplinks are activated.

Table 5.1.4.2-1: UE RF Rx requirements for dual uplink inter-band CA

| Section / Clause in TS36.101 | Description | Comments |
| --- | --- | --- |
| 7.3.1A | Reference sensitivity for CA | Need to be tested for all x bands DL/ 2 bands UL |
| 7.6.2.1A | Out-of-band blocking for CA | No need to be tested for x bands DL/ 2 bands UL |
| 7.7.1A | Spurious response for CA | No need to be tested for x bands DL/ 2 bands UL |

5.2	RRM part

[Editor Note] It will be updated in future

<End of TP>