3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010335

# Electronic Meeting, 17 Aug. – 28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0996 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction, applicablity and capability for CSI-RS inter-frequency measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the defintion of inter-frequency measurement for L3 CSI-RS based measurements, and specify requirement applicability and number of cells/beams to be measured for inter-frequency measurement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the agreements on CSI-RS inter-frequency measurement in RAN4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CSI-RS based inter-frequency measurement is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

9.x.y	CSI-RS based Inter-frequency measurements

9.x.y.1	Introduction

A measurement is defined as an CSI-RS based inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.x.z.

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and the higher layer parameter associatedSSB is configured, the UE shall be able to identify inter-frequency cells indicated for measurment and perform CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements of identified inter-frequency cells.

[CSI-RS based measurements are configured along with a measurement timing configuration (SMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements of CSI-RS on the configured inter-frequency carrier are to be performed. For inter-frequency connected mode measurements, one measurement window periodicity may be configured per inter-frequency measurement object.]

When measurement gaps are needed, the UE is not expected to detect the associated SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor perform measurement of the CSI-RS resource configured in CSI-RS-Resource-Mobility which ends later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE in EN-DC, SA NR, NE-DC and NR-DC, or the serving cells are in FR2, the inter-frequency cells are in FR2 and the per-UE gap is configured to the UE in SA NR and NR-DC, the switching time is 0.25ms. Otherwise the switching time is 0.5ms.

9.x.y.2	Requirements applicability

The requirements in clause 9.x.y apply, provided:

-	all CSI-RS resources in the same MO is configured with the same csi-rs-MeasurementBW.

-	The associated SSB of the cell being identified or measured is detectable.

-	The CSI-RS for measurement is QCL-ed to the associated SSB for FR2.

9.x.y.3	Number of cells and number of CSI-RS resources

9.x.y.3.1	Requirements for FR1

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-	4 identified cells, and

-	7 CSI-RSs with different associated SSB index and/or PCI on the inter-frequency layer.

9.x.y.3.2	Requirements for FR2

For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least:

-	4 identified cells, and

-	14 CSI-RSs with different associated SSB index and/or PCI on the inter-frequency layer.

<< End of change 1>>