3GPP TSG-RAN WG4 Meeting # 96-e                	R4-2010388

Electronic Meeting, Aug. 17-28, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on synchronization assumption

Agenda item:	7.14.1.5

Document for:	Discussion

Introduction

During RAN#95-e meeting, the synchronization assumption for the CSI-RS based measurement was discussed but was not concluded due to diverse understandings. The way forward is captured in [1] with several options as below. In this paper, we are discussing the synchronization assumption under the agreed CSI-RS configurations in Rel16.

  - For CSI-RS measurement with associated SSB,

  - Option 1: RAN4 to address the issue of timing difference between the arrival of the CSI-RS and UE’s FFT timing in the performance part.

  - Option 2: the corresponding timing of CSI-RS resources should be assumed the same as the timing of the cell given by the cellId of the CSI-RS resource configuration.

  - Option 3: introduce the UE capability to differentiate the following 2 types of UEs.

    - Type 1: UE supporting using only single timing for CSI-RS measurement per frequency layer

    - Type 2: UE supporting using timing of associated SSB for respective CSI-RS measurement

Discussion

In last meeting, it was agreed to define the CSI-RS based measurement requirement in Rel16 within the scope below. In particular, the requirements are defined only when associatedSSB is configured and detected by the UE.

Scope of Requirements

- Define requirements only for intra-f without gap and inter-f with gap in Rel-16.

- No requirements shall be defined in Rel-16 for CSI-RS L3 measurement, when

    - associated SSB is not configured

    - associated SSB is not detected even if associated SSB is configured

- UE is not required to perform CSI-RS based L3 measurement when associated SSB is not QCLed with CSI-RS for FR2

According to 3GPP TS 38.214, if associatedSSB is configured, the UE is required to detect the associatedSSB for the timing of the cell before it can monitor the corresponding CSI-RS resource. In this case, the cell to be measured does not have to be synchronized with the serving cell. That is, tight cell synchronization is not always assumed when defining the CSI-RS based measurement requirements in Rel16.

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and the higher layer parameter associatedSSB is not configured, the UE shall perform measurements based on CSI-RS-Resource-Mobility and the UE may base the timing of the CSI-RS resource on the timing of the serving cell.

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE may base the timing of the CSI-RS resource on the timing of the cell given by the cellId of the CSI-RS resource configuration. Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, the UE is not required to monitor the corresponding CSI-RS resource. The higher layer parameter isQuasiColocated indicates whether the associated SS/PBCH block given by the associatedSSB and the CSI-RS resource(s) are quasi co-located with respect to ['QCL-TypeD'].

Observation#1: If associatedSSB is configured, tight cell synchronization is not always assumed between the cell to be measured and the serving cell.

Meanwhile in the WID of the CSI-RS based RRM requirements [2], there are assumptions on synchronization that single FFT shall be applied when performing multiple cell measurements on one carrier. It is understood the single FFT is used to enable simultaneous process of CSI-RSs from multiple neighbor cells on one carrier. As no synchronization is assumed among the cells, there would be timing difference between the CSI-RS resources to be measured and the timing of the single FFT, which may lead to the performance degradation. The impact due to the timing difference needs to be considered under the single FFT assumption when defining the requirements in Rel16.

This new WID proposed to specify RRM requirements for CSI-RS based L3 measurements with the following aspects,

- Assumptions on Synchronization

  - Single FFT is assumed for multiple cell measurements per frequency layer for both intra- and inter-frequency measurements.

Observation#2: There may be timing difference between the CSI-RS resources to be measured and the timing of single FFT.

Proposal#1:  The impact due to the timing difference needs to be considered under the single FFT assumption when defining the requirements in Rel16.

We have simulated the performance under 3us timing difference considering the maximum cell phase error. In practice, the timing difference may also be impacted due to propagation delay, timing advance etc. The results at different SCSs are shown in Table 1. It was seen that the performance at lower SCS e.g. 15kHz and 30kHz was slightly degraded but still at an acceptable level, as 3us can still be fit into one CP length. However, at higher SCS i.e. 60kHz and 120kHz, the CP length is very short i.e. 1.17us and 0.57us respectively. The 3us timing difference would span over 2 and 5 times of CP length, where the measurement performance is degraded significantly and becomes insensible. The CSI-RS based measurement result makes sense only when the timing difference is within the CP length.

Observation#3: The CSI-RS based measurement result makes sense only when the timing difference is within the CP length.

Table 1. Performance under 3us timing difference for respective SCSs

As the timing difference issue occurs because of the single FFT, the UE capability can be defined to indicate whether single FFT is applied when performing the CSI-RS based measurement on one carrier.

Proposal2: UE capability can be defined to indicate whether single FFT is applied when performing the CSI-RS based measurement on one carrier.

Conclusion

This contribution identifies the conflicts between the assumptions of synchronization in the WID and the assumption behind the configuration of associatedSSB in Rel16.  It is proposed the timing difference impact due to single FFT shall be considered when defining the Rel16 requirements.

Observation#1: If associatedSSB is configured, tight cell synchronization is not always assumed between the cell to be measured and the serving cell.

Observation#2: There may be timing difference between the CSI-RS resources to be measured and the timing of single FFT.

Proposal#1:  The impact due to the timing difference needs to be considered under the single FFT assumption when defining the requirements in Rel16.

Observation#3: The CSI-RS based measurement result makes sense only when the timing difference is within the CP length.

Proposal2: UE capability can be defined to indicate whether single FFT is applied when performing the CSI-RS based measurement on one frequency layer.

References

- R4-2009256, WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement