3GPP TSG-RAN WG4 Meeting #96-e	R4-2010472

Electronic Meeting, 17 – 28 August 2020

Agenda Item:	6.1.3.2

Source:	Ericsson

Title:	Simulation results of CSI-RS based PMI reporting test

Document for:	Discussion

# 1	Introduction

RAN4#95-e agreed with the simulation assumption of CSI-RS based PMI reporting test for non-BL UE [1].

CSI reporting requirements due to the introduction of CSI-RS based feedback - Set γ=1.2 (Throughput ratio of follow PMI over random PMI) for CSI-RS based PMI reporting test for non-BL UE at the SNR where 70% of the maximum throughput is achieved with the follow PMI. - Interested companies bring simulation results in RAN4#96-e to decide the SNR test points for both FDD and TDD. - Refer to R4-2007375 for simulation assumption

# 2	CSI-RS based PMI reporting test

## 2.1	Simulation parameters

Table 1 and Table 2 show the simulation parameters and FRC used for the simulation according to [2]

Table 1	Simulation parameters for CSI-RS based CSI reporting tests.

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| PDSCH transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 6 |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 |
|  | σ | dB | -3 |
|  | δ | dB | 0 |
| CRS reference signals |  |  | Antenna ports 0, 1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |
| CSI reference signal configuration |  |  | 0 |
| Propagation condition and antenna configuration |  |  | High XP 8 x 2 |
| Beamforming Model |  |  | As specified in TS36.101 B.4.3 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 001F FFE0 0000 0000 FFFF |
|  |  | dB[mW/15kHz] | -98 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| Reporting periodicity |  | ms | 10 |
| PMI delay |  | ms | 10 |
| cqi-pmi-ConfigurationIndex |  |  | 12 |
| ce-csi-rs-feedback-config |  |  | Configured |
| Frequency hopping |  |  | Disabled |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |
| PDSCH repetition level |  |  | 1 |
| MPDCCH repetition level |  |  | 1 |
| Beamforming Precoder for MPDCCH |  |  | No precoding |
| Precoder update granularity for MPDCCH |  |  | N/A |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |
| PDSCH PRB size |  | PRB | 3 |
| PDSCH MCS |  |  | QPSK 1/2 |
| RI |  |  | 1 |

Table 2	RMC for CSI-RS based PMI reporting test.

| Parameter | Unit | Values |
| --- | --- | --- |
| Allocated resource blocks |  | 4 |
| Modulation |  | QPSK |
| Target Coding rate |  | 1/2 |
| Information bit payload for non CSI-RS subframe | bits | 472 |
| Information bit payload for CSI-RS subframe | bits | 472 |
| Binary channel bits for non CSI-RS subframe | bits | 896 |
| Binary channel bits for non CSI-RS subframe | bits | 960 |

## 2.2	Simulation results

Figure 1 (a) and (b) compare the relative throughputs with follow PMI and random PMI for FDD and TDD, and Figure 2 shows the throughput ratio of follow PMI and random PMI. Table 3 summarizes the throughput ratio of follow PMI and random PMI at SNR achieving the 70% of the maximum throughput with follow PMI.

Observation: Throughput ratio of 1.4 is achieved with 70% of maximum throughput with follow PMI.

Considering the implementation margin, we propose to confirm the throughput ratio of follow PMI over random PMI (γ) to 1.2, for the CSI-RS based PMI reporting test for non-BL UE at SNR where 70% of the maximum throughput is achieved with follow PMI, for both FDD and TDD.

Proposal: Confirm the throughput ratio of follow PMI over random PMI (γ) to 1.2, for the CSI-RS based PMI reporting test for non-BL UE at SNR where 70% of the maximum throughput is achieved with follow PMI, for both FDD and TDD.

| (a) FDD | (b) TDD |
| --- | --- |

Figure 1	Comparison of follow PMI and random PMI.

Figure 2	Throughput ratio of follow PMI and random PMI.

Table 3	Throughput ratio of follow PMI and random PMI at SNR achieving 70% of maximum throughput with follow PMI.

| SNR (dB) | SNR to achieve 70% of maximum throughput with follow PMI | Throughput ratio of follow PMI and random PMI |
| --- | --- | --- |
| FDD | -2.3 | 1.44 |
| TDD | -2.3 | 1.46 |

# 3	Summary

Observation: Throughput ratio of 1.4 is achieved with 70% of maximum throughput with follow PMI.

Proposal: Confirm the throughput ratio of follow PMI over random PMI (γ) to 1.2, for the CSI-RS based PMI reporting test for non-BL UE at SNR where 70% of the maximum throughput is achieved with follow PMI, for both FDD and TDD.

# 4	References

- R4-2008758, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”, Ericsson.

- R4-2005513, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”, Ericsson.