3GPP TSG-RAN WG4 Meeting #96-e								          R4-2010498

Online, 17th – 28th August 2020

Source: 	Huawei, HiSilicon

Title: 	Spectrum utilization for NR-U

Agenda Item:	7.1.1.2

Document for:	Approval

Introduction

In this contribution, we discuss the minimum guardband for wideband operation in NR-U.

Discussion

In RAN4 #92 meeting, we have agreed to increase the number of PRBs to 25 for 20 MHZ channel bandwidth with 60 kHz SCS with the condition of relaxation on NR-U emission requirements compared with R15 NR. Since the UE Tx requirement for PC5 was agreed in last meeting with the relaxed emission requirement compared with R15 NR, 25PRB for 20 MHZ channel bandwidth should be mandatory for a UE supporting 60 kHz SCS.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 241 252 | 31 | 38 | 51 | 65 | 79 | 93 | 107 | 121 | 135 |
| Note 1: It is not applied for Band n46. Note 2: It is only applied for Band n46. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: 25PRB for 20 MHZ channel bandwidth should be mandatory for a UE supporting 60 kHz SCS.

In RAN4 #93 meeting，the following WF was achieved:

- The default pattern is that for which the radio requirements apply given the specified channel raster points, starting at the first usable PRB, for usable PRBs and Intra-carrier guard PRBs are given for different SCS and CBW in the following table together with maximum number of RBs:

| SCS | 20MHz Channels | 40MHz Channels |  | 60MHz Channels |  | 80MHz Channels |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz | 106 | 105-6-105 | Max. 216 | N/A |  | N/A |  |
| 30KHz | 51 | 50-6-50 | Max. 106 | 50-6-50-6-50 | Max. 162 | 50-6-50-5-50-6-50 | Max. 217 |
| Alt. 1 60KHz | 24 | [23-5-23] | Max. 51 | [23-5-23-5-23] | Max. 79 | [23-5-23-5-23-5-23] | Max. 107 |
| Alt. 2 60KHz | [25] | [24-3-24] | Max. 51 | [24-3-25-3-24] | Max. 79 | [24-4-24-3-24-4-24] | Max. 107 |

We still need to define the default pattern for cases with 60kHz SCS.  Since we have already agreed to improve the SU to 25PRB for 20MHz with 60kSCS in NR-U, which means that the minimum guardband for 20MHz carrier should be reduced, which will correspondingly result in a smaller intra-carrier guardband, and Alt.2 in above table should be supported.

Proposal 2: Alt.2 for 60kHz intra-carrier guardbands should be supported.

Conclusions

In this contribution, we discussed the minimum guardband for wideband operation in NR-U, and the following proposals were made:

Proposal 1: 25PRB for 20 MHZ channel bandwidth should be mandatory for a UE supporting 60 kHz SCS.

Proposal 2: Alt.2 for 60kHz intra-carrier guardbands should be supported.

References

RAN4#93 Meeting report, Reno, USA, Nov 18-22 2019

R4-1916160 WF on guard-band design for NR-U

38.104 V16.0.0  Base Station (BS) radio transmission and reception