3GPP TSG-RAN WG 4 Meeting #96-e	R4-2010552

# Online, 17th – 28th August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to 38101-1 on CA_n41C_SUL_n41A-n80A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, HiSilicon, MediaTek, Spreadtrum |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SUL_combos_R17 |  |  |  |  |  | Date: |  |  | 2020-07-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | This draftCR is to introduce the band combination of CA_n41C_SUL_n41A-n80A according to operators’ requests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify CA_n41C_SUL_n41A-n80A UE requirements by reusing the ones for SUL_n41A-n80A. The UL configurations of the both combinations are the exact same. For DL, no spec impact is needed compared to SUL_n41A-n80A for CA_n41C_SUL_n41A-n80A. No specific requirements are specified in addition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band combination is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2C, 5.5C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change1>>

## 5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1 and Table 5.2C-2, where all operating bands are within FR1.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n80 | n41, n80 |
| SUL_n41-n81 | n41, n81 |
| SUL_n41-n95 | n41, n95 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n84 | n79, n84 |
| SUL_n79-n95 | n79, n95 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: For UE supporting SUL band combination, UL MIMO is not configured on SUL carrier |  |

Table 5.2C-2: Operating SUL band combination with downlink CA in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n78(*)_SUL_n78-n862 | n78, n86 |
| CA_n41C_SUL_n41-n80 | n41, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. NOTE 3: For UE supporting SUL band combination, UL MIMO is not configured on SUL carrier. NOTE 4: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in table 5.5C-2. |  |

## <<End of Change1>>

## <<Start of Change2>>

## 5.5C	Configurations for SUL

Table 5.5C-1: Supported channel bandwidths per SUL band combination

| SUL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n80A | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| SUL_n41A-n81A | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n81 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| SUL_n41A-n95A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n95 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| SUL_n77A-n80A | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| SUL_n77A-n84A | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n80A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| SUL_n78A-n81A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n82A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n82 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n83A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n83 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n84A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n78A-n86A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n86 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n80A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| SUL_n79A-n81A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  | n81 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n84A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  | n84 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| SUL_n79A-n95A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  | n95 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |

Table 5.5C-2: Supported channel bandwidths per SUL band combination with downlink CA

| SUL band combination with downlink CA | SUL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78(2A)_SUL_n78A-n86A | SUL_n78A-n86A | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n86 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| CA_n41C_SUL_n41A-n80A | SUL_n41A-n80A | n41 | See CA_n41C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

## <<End of Change2>>