3GPP TSG-RAN WG4 Meeting#96                	R4-2010561

E-meeting, August 17th – 28th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Search threshold applicability in euCA

Agenda item:	5.3

Document for:	Discussion

Introduction

In RAN4#93 Reno, it was agreed to align the RAN4 euCA requirements with the RAN2 decision stating the inter-frequency search thresholds (s-NonIntraSearch) are not applied for carriers configured for early measurement reporting. The CR was agreed in [2]. In this paper we address this topic.

Discussion

In euCA, the network can configure the UE with both overlapping and non-overlapping carriers for UE to measure for early reporting. For overlapping carriers, the requirements are the same as for carriers used for mobility with the exception that s-NonIntraSearch thresholds do not apply.

Not applying the search thresholds EMR measurements performed on carriers configured for EMR measurements was just confirmed by RAN2 in their reply LS [8].

As the RAN4 requirements in 36.133 section 4.9.2.2 refers to requirements in section 4.2.2.4, in which the s-NonIntraSearch may be applied, the current requirements are unclear. We propose to clarify in the RAN4 specification 36.133 section 4.9.2.2, that the s-NonIntraSearch thresholds do not apply to carriers configured for early measurement reporting.

Clarify in 36.133 that the fields s-NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells.

Conclusion

In RAN4 it was agreed to align the RAN4 euCA requirements with the RAN2 decision stating that search thresholds (s-NonIntraSearch) do not apply to carriers configured for early measurement reporting. The CR was endorsed in [2]. RAN2 has also confirmed similar behaviour when discussing the MR-DC EMR work.

We propose:

- Clarify in 36.133 that the fields s-NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells.

In [9] we have provided CR.

References

- R4-1914463, Discussion on number of carriers to measure in euCA, Nokia, Nokia Shanghai Bell

- R4-1912845, Correction to S-measure threshold applicability in IDLE mode measurements, Nokia, Nokia Shanghai Bell

- R4-1914463, Discussion on number of carriers to measure in euCA, Nokia, Nokia Shanghai Bell

- R4-1914464, CR on Number of carriers to monitor for IDLE mode measurements, Nokia, Nokia Shanghai Bell

- R4-1915554, Correction to S-measure threshold applicability in IDLE mode measurements, Nokia, Nokia Shanghai Bell

- R4-2007147, CR on Number of carriers to monitor for IDLE mode measurements, Nokia, Nokia Shanghai Bell

- R4-2007149, CR clarifying S-measure thresholds for EMR carriers, Nokia, Nokia Shanghai Bell

- R2-2006287, Reply LS RAN4 on RRM Enhanced Measurement Reporting, RAN2

- R4-2010562, CR clarifying S-measure thresholds for EMR carriers, Nokia, Nokia Shanghai Bell