3GPP TSG-RAN WG4 Meeting # 96-e                                                           R4-2010648

# Electronic Meeting, 17-28 Aug., 2020

Source:	ZTE Corporation

Title:	Emission for IAB EMC

Agenda Item:	7.4.4.2

Document for:	Approval

- Introduction

During the RAN4#95-e meeting, it was agreed  to have a new TS for IAB EMC according to WF[1].

There are some discussions on the TS skeleton and work splits for IAB EMC before the meeting. Based on the discussions, the TS skeleton is stable as in [2].

In this contribution, we discuss IAB emission requirements for IAB EMC and provide a TP for the specification based on [2].

- Discussion

The agreements for radiated emission has been captured in the WF[1] which is shown below:

Emission requirement

For Radiated Emission requirement, wait to see the RF spurious emission outcome for further study.

For conducted emission requirement, it is agree to reuse the base station frame work and the wording are per requirement basis.

Based on the above, the BS RF conducted emission requirements shall be reused for IAB EMC conducted emission requirements.

For the radiated emission, although there are no consensus for RF spurious emission, generally the spurious emission requirements are based on the ITU-R SM.329.

In current TS 38.113[3], the radiated emission limits for NR BS type 1-C and BS type 1-H are defined in Table 8.2.1.3-1 shown in the follow:

Table 1: Limits for NR BS type 1-C and BS type 1-H radiated emissions (cited form table 8.2.1.3-1 in TS 38.113 )

| Frequency Range | Minimum requirement (e.r.p.) / Reference bandwidth | NOTE |
| --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm/ 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm/ 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm/ 1 MHz | 1 |
| FDL,low - ΔfOBUE < f < FDL,high +ΔfOBUE | Not defined | 2,3 |
| NOTE 1: This frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 2: For BS capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 3: ΔfOBUE is defined in subclause 6.6.1 of TS 38.104 [2]. |  |  |

In addition, the radiated emission limits for NR UE operations in FR1 are defined in Table 8.2.4-1 in TS 38.124[4], and the boundary for the exclusion frequency ranges are defined in table 8.2.4-0 which is FOOB from the edge of the channel bandwidth, shown in the follow tables.

Table 2. Radiated spurious emissions requirements for UE equipment supporting operations in FR1 only (cited form Table 8.2.4-1 in TS 38.124)

| Frequency range | Maximum level (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 | 1 MHz |  |
|  | -25 | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 | 1 MHz | 2 |
| NOTE 1: Applies for Band that the upper frequency edge of the UL Band more than 2.69 GHz. NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz. NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

Table 3: Boundary between NR out of band and general spurious emission domain (cited from Table 8.2.4-0 in TS 38.124 )

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| BWChannel | BWChannel + 5 |

For EMC radiated emission, the test is applicable to BS type 1-C and BS type 1-H. For BS type 1-O and BS type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 38.104, and conforming to the test requirement in TS 38.141-2.

It can be seen from table 2, the radiated emissions emission for UE EMC and BS EMC are almost the same. Actually it comes from ITU-R SM.329. It shall be noted for UE EMC, the frequency range should be from 30MHz for radiated emission and the exclusive areas are different for UE and BS.

For the last column in table 2, the frequency range is ‘12.75 GHz < f < 26 GHz’, which is applied to band that the upper frequency edge of the UL Band more than 5.2 GHz. According to TS38.174, the operating bands are designed for NR IAB are shown in table 4 as follow..

Table 4:NR IAB operating bands in FR1(cited from Table 5.2-1 in TS 38.174 )

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- | --- |
| n41 | 2469 MHz – 2690 MHz | 2469 MHz – 2690 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

It can be seen that all of the NR IAB in FR1 operating are all TDD mode. The uplink operating bands are the same as downlink operation band, and the highest frequency is 5000MHz (i.e. band n79), the 5th harmonic equals to 25G, which means UE EMC and BS EMC are the same.

Considering the possible different implementations for IAB MT and IAB DU, where the MT and DU may be deployed in one enclose, or in different enclosures. No matter which implementations, the IAB MT and IAB DU should be tested at the same time with different link configuration. For the radiated emission, we propose to use a common table to cover the requirements for both IAB MT and IAB DU, shown in table 5.

Table 5.  Limits for radiated emissions for IAB

| Frequency Range | Minimum requirement (e.r.p.) / Reference bandwidth | NOTE |
| --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm/ 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm/ 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm/ 1 MHz | 1 |
| FDL,low - ΔfOBUE < f < FDL,high +ΔfOBUE | Not defined | 2,3 |
| FUL,low - FOOB < f < FUL,high + FOOB | Not defined | 4,5 |
| NOTE 1: This frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 2: For IAB-DU capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 3: ΔfOBUE is defined in subclause 6.6.1 of TS 38.104. NOTE 4: For IAB-MT capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 5: FOOB is defined in subclause 6.5.3 of TS 38.101-1. |  |  |

- Conclusion

In this paper, we give some discussion on IAB EMC emission requirement.

For conduct emission requirements, the BS RF conducted emission requirements shall be reused.

For radiated emission requirements, only type 1-H is considered. Although there are no consensus for RF spurious emission, generally the spurious emission requirements are based on the ITU-R SM.329. Considering the spurious emission requirements for UE and BS are almost the same, we propose to to use a common table to cover the requirements for both IAB MT and IAB DU.

- Reference

- R4-2008732, WF on IAB EMC, ZTE

- R4-2010647, Proposal on the skeleton of  IAB EMC, ZTE

- TS 38.113, NR; Base Station (BS) ElectroMagnetic Compatibility (EMC)

- TS 38.124, NR; ElectroMagnetic Compatibility (EMC) requirements for mobile terminals and ancillary equipment

- TS 38.174, NR; Integrated access and backhaul radio transmission and reception

- Text Proposal

----- Start of TP -----

- 8	Emission

- 8.1	Test configurations

Texts will be added.

- 8.2	Radiated emission

- 8.2.1	Radiated emission, IAB

This test is applicable to IAB type 1-H. This test shall be performed on a representative configuration of IAB.

For IAB type 1-O and IAB type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 38.174 [11], conforming to the test requirement in TS 38.174[11].

- 8.2.1.1	Definition

This test assesses the ability of IAB to limit unwanted emission from the enclosure port.

- 8.2.1.2	Test method

a)	A test site fulfilling the requirements of ITU-R SM.329 [25] shall be used. The IAB shall be placed on a non-conducting support and shall be operated from a power source via a RF filter to avoid radiation from the power leads.

Mean power of any spurious components shall be detected by the test antenna and measuring receiver (e.g. a spectrum analyser). At each frequency at which a component is detected, the IAB shall be rotated and the height of the test antenna adjusted to obtain maximum response, and the effective radiated power (e.r.p.) of that component determined by a substitution measurement. The measurement shall be repeated with the test antenna in the orthogonal polarization plane.

NOTE:	Effective radiated power (e.r.p.) refers to the radiation of a half wave tuned dipole instead of an isotropic antenna. There is a constant difference of 2.15 dB between e.i.r.p. and e.r.p, as defined in ITU-R SM.329 annex 1 [25].

e.r.p. (dBm)  EIRP (dBm)  2.15

b)	The IAB shall transmit with maximum power declared by the manufacturer with all transmitters active. Set the base station to transmit a signal as stated in subclause 4.5.

- 8.2.1.3	Limits

Unless otherwise stated, the radiated spurious emission limits apply for the frequency ranges that are as the following:

-	more than FOOB (MHz) in table 8.2.1.3-1 from the edge of the channel bandwidth for IAB-MT. The radiated spurious emission limits in table 8.2.1.3-1 apply for all transmitter band configurations (NRB) and channel bandwidths of IAB-MT.

-	more than ΔfOBUE(MHz) in table 8.2.1.3-1 from the edge of the downlink operation band for IAB-DU.For IAB-DU capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply.

-	more than the combined exclusion frequency ranges for IAB-MT and IAB-DU while IAB-DU and IAB-MT are together in one enclosure case.

The frequency boundary and reference bandwidths for the detailed transitions of the limits between the requirements for out of band emissions and spurious emissions are based on ITU-R Recommendations SM.329 [25] and SM.1539 [26]. Unless otherwise stated, all measurements are done as mean power (RMS).

The IAB type 1-H shall meet the limits below:

Table 8.2.1.3-1: Limits for radiated emissions for IAB

| Frequency Range | Minimum requirement (e.r.p.) / Reference bandwidth | NOTE |
| --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm/ 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm/ 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm/ 1 MHz | 1 |
| FDL,low - ΔfOBUE < f < FDL,high +ΔfOBUE | Not defined | 2,3 |
| FUL,low - FOOB < f < FUL,high + FOOB | Not defined | 4,5 |
| NOTE 1: This frequency range applies only for operating bands for which the 5th harmonic of the upper frequency edge of the DL operating band is reaching beyond 12.75 GHz. NOTE 2: For IAB-DU capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 3: ΔfOBUE is defined in subclause 6.6.1 of TS 38.104 [6]. NOTE 4: For IAB-MT capable of multi-band operation, the frequency ranges relating to the RF bandwidths of all supported operating bands apply. NOTE 5: FOOB is defined in subclause 6.5.3 of TS 38.101-1 [3]. |  |  |

- 8.2.1.4	Interpretation of the measurement results

The interpretation of the results recorded in a test report for the radiated emission measurements described in the present document shall be as follows:

-	the measured value related to the corresponding limit will be used to decide whether an equipment meets the requirements of the present document;

-	the value of the measurement uncertainty for the measurement of each parameter shall be included in the test report;

-	the recorded value of the measurement uncertainty shall be, for each measurement, equal to or lower than the figures in table 8.2.1.4-1 for IAB.

Table 8.2.1.4-1 specifies the maximum measurement uncertainty of the test system. The test system shall enable the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95% of the performance of a population of test equipment.

Table 8.2.1.4-1: Maximum measurement uncertainty (IAB)

| Parameter | Uncertainty for EUT dimension ≤ 1 m | Uncertainty for EUT dimension >1 m |
| --- | --- | --- |
| Effective radiated RF power between 30 MHz to 180 MHz | 6 dB | 6 dB |
| Effective radiated RF power between 180 MHz to 4 GHz | 4 dB | 6 dB |
| Effective radiated RF power between 4 GHz to 12,75 GHz | 6 dB | 9 dB (NOTE) |
| NOTE: This value may be reduced to 6 dB when further information on the potential radiation characteristic of the EUT is available. |  |  |

NOTE:	If the test system for a test is known to have a measurement uncertainty greater than that specified in table 8.2.1.4-1, this equipment can still be used, provided that an adjustment is made follows:

Any additional uncertainty in the test system over and above that specified in table 8.2.1.4-1 is used to tighten the test requirements - making the test harder to pass.

This procedure will ensure that a test system not compliant with table 8.2.1.4-1 does not increase the probability of passing an EUT that would otherwise have failed a test if a test system compliant with table 8.2.1.4-1 had been used.

- 8.2.2	Radiated emission, ancillary equipment

This test is only applicable to ancillary equipment not incorporated in the radio equipment and intended to be measured on a stand-alone basis, as declared by the manufacturer. This test shall be performed on a representative configuration of the ancillary equipment.

This test is not applicable for ancillary equipment incorporated in the radio equipment, or for ancillary equipment intended to be measured in combination with the radio equipment. In these cases, the requirements of the relevant product standard for the effective use of the radio spectrum shall apply.

- 8.2.2.1	Definition

This test assesses the ability of ancillary equipment to limit unwanted emission from the enclosure port.

- 8.2.2.2	Test method

The test method shall be in accordance with CISPR 32 12.

- 8.2.2.3	Limits

The ancillary equipment shall meet the limits according to CISPR 32 12 table A.4 and table A.5.

For the referred limit values, the following shall apply:

Where the limits value varies over a given frequency range, it changes linearly with respect to the logarithm of the frequency.

Where there is a step in the relevant limit, the lower value shall be applied at the transition frequency.

- 8.3	Conducted emission DC power input/output port

If the DC power cable of the radio equipment is intended to be less than 3 m in length, and intended only for direct connection to a dedicated AC to DC power supply, then the measurement shall be performed only on the AC power input of that power supply as specified in subclause 8.4.

This test shall be performed on a representative configuration of the radio equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

- 8.3.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to limit internal noise from the DC power input/output ports.

- 8.3.2	Test method

The test method shall be in accordance with CISPR 32 [12] and the Artificial Mains Network (AMN) shall be connected to a DC power source.

In the case of DC output ports, the ports shall be connected via an AMN to a load drawing the rated current of the source.

A measuring receiver shall be connected to each AMN measurement port in turn and the conducted emission recorded.

The equipment shall be installed with a ground plane as defined in CISPR 32 [12]. The reference earth point of the AMN shall be connected to the reference ground plane with a conductor as short as possible.

- 8.3.3	Limits

The equipment shall meet the limits according to CISPR 32 [14] table A.9, which are defined for average detector receiver and for quasi-peak detector receiver. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

Where there is a step in the referred limit values, the lower value shall be applied at the transition frequency.

- 8.4	Conducted emissions, AC mains power input/output port

This test is applicable to equipment powered by the AC mains.

This test is not applicable to AC output ports which are connected directly (or via a circuit breaker) to the AC power port of the EUT.

This test shall be performed on a representative configuration of the radio equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

- 8.4.1	Definition

This test assesses the ability of radio equipment and ancillary equipment to limit internal noise from the AC mains power input/output ports.

- 8.4.2	Test method

The test method shall be in accordance with CISPR 32 [12].

- 8.4.3	Limits

The equipment shall meet the limits according to CISPR 32 [12] table A.10, which are defined for the average detector receiver and for quasi-peak detector receiver. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

For the referred limit values following shall apply:

Where the limits value varies over a given frequency range, it changes linearly with respect to the logarithm of the frequency.

Where there is a step in the relevant limit, the lower value shall be applied at the transition frequency.

Alternatively, for equipment intended to be used in telecommunication centres the limits given in CISPR 32 [12] table A.9 shall be used.

- 8.5	Conducted emissions, telecommunication port

This test is applicable for radio equipment and/or ancillary equipment for fixed use which have telecommunication ports.

This test shall be performed on a representative configuration of radio equipment, the associated ancillary equipment, or a representative configuration of the combination of radio and ancillary equipment.

- 8.5.1	Definition

This test assesses the EUT unwanted emission present at the telecommunication ports.

- 8.5.2	Test method

The test method shall be in accordance with CISPR 32 [12].

- 8.5.3	Limits

The telecommunication ports shall meet the limits according to CISPR 32 [12] table A.12.

For the referred limit values, following shall apply:

Where the limits value varies over a given frequency range, it changes linearly with respect to the logarithm of the frequency.

Where there is a step in the relevant limit, the lower value shall be applied at the transition frequency.

Alternatively, for equipment intended to be used in telecommunication centres only, the limits given in CISPR 32 [12] table A.11 may be used.

- 8.6	Harmonic Current emissions (AC mains input port)

The requirements of IEC 61000-3-2 [14] for harmonic current emission apply for equipment covered by the scope of the present document. For equipment with an input current of greater than 16A per phase, IEC 61000-3-12 [15] applies.

- 8.7	Voltage fluctuations and flicker (AC mains input port)

The requirements of IEC 61000-3-3 [16] for voltage fluctuations and flicker apply for equipment covered by the scope of the present document. For equipment with an input current of greater than 16 A per phase, IEC 61000-3-11 [17] applies.

----- End of TP -----