3GPP TSG-RAN WG4 Meeting #96-e	R4-2010697

Electronic Meeting, 17 August – 28 August 2020

Source:	Ericsson, US Cellular

Title:	TP for TR 37.717-21-11: Including band combinations DC_2-12_n5

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_2A-12A_n5A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_2-12_n5

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12_n5 | CA_2-12 | n5 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A_n5A | DC_2A_n5A DC_12A_n5A | DC_2A-12A | n5A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_2-12_n5, the TIB,c and RIB,c values are reused from LTE combination CA_2-5-12 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n5 | 2 | 0.3 |
|  | 12 | 0.4 |
|  | n5 | 0.8 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n5 | 2 | 0 |
|  | 12 | 0.3 |
|  | n5 | 0.5 |

### 5.1.x.4	REFSENS requirements

There is IMD5 impact from DC_12_n5 UL affecting band 2 DL. MSD derived from DC_7A-28A_n5A and DC_20A_n7A-n28A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-12A_n5A | 2 | 1886 | 5 | 25 | 1966 | 5.9 | IMD5 |
|  | 12 | 700 | 5 | 25 | 730 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-201369, “New WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon