3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010704

# Electronic Meeting, 17 – 28 Aug., 2020

Source:	OPPO

Title:	Discussion on RRM requirements maintenance of measurement relaxation for UE power saving

Agenda Item:	7.6.2

Document for:	Discussion

Introduction

In RAN4 #95e meeting, a WF [1] and a CR [2] on IDLE state measurement relaxation for UE power saving were approved. Unfortunately, due to the limited time for reviewing the agreements of RRM measurements relaxation for power saving were not correctly captured.

In this contribution, we further clarify the requirements maintenance of measurement relaxation for UE power saving.

Discussion

For IDLE state measurement relaxation for UE power saving, the following applicable RRM relaxation methods were agreed in last several meetings.

| Scenario | Applicable RRM relaxation methods |
| --- | --- |
| Scenarios #1: Low mobility scenario | Option 1: Allow RRM measurements with longer intervals - The scaling factor (K1=3) of RRM measurement relaxation with longer intervals shall apply to Tdetect, Tevaluate, and Tmeasure. |
| Scenarios #2: Not in cell-edge scenario | Option 1: Allow RRM measurements with longer intervals The scaling factor (K1=3) of RRM measurement relaxation with longer intervals shall apply to Tdetect, Tevaluate, and Tmeasure. |
| Scenarios #3: Low-mobility and Not in cell-edge scenario | Option 2: UE is not required to meet the intra-frequency and inter-frequency neighbor cell measurement requirements - The time interval for measurement relaxation since last measurement for cell reselection for scenario#3 is 1 hour |

According to RAN4’s agreements, in summary, RRM measurement relaxation for inter-frequency and inter-RAT layer with higher priority shall follow the principles as follows.

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the relaxed requirement for the frequency layer of higher priority shall use the same relaxed measurement requirement as those for the frequency layer of equal/lower priority.

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, no relaxation of the current measurement delay requirement is expected for inter-frequency measurement with higher priority, if scenario 2 (Not in cell-edge scenario) is fulfilled and condition of scenario 1 is not fulfilled or not configured. AndUE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements in scenario 3.

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and only criteria of low mobility is configured, if the low mobility criteria is fulfilled

  - UE is not required to do any lower and equal priority inter-frequency/inter-RAT measurement

  - When NW indicates that higher priority carrier measurements can be relaxed (highPriorityMeasRelax), UE measures higher priority inter-frequency/inter-RAT layers at least every 1 hour

  - When NW does not indicate that higher priority carrier measurements can be relaxed, UE measures higher priority inter-frequency/inter-RAT layers at least every Thigher_priority_search (60 sec).

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, if criteria of low mobility and not in cell edge are both fulfilled,

  - UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements

    - 1 hour time interval applies to the measurement relaxation since last measurement for cell reselection for higher priority carriers

Thus, we have the observation that for inter-frequency / inter-RAT frequency measurement, RRM measurement relaxation shall be meet by UE as Table 1:

Proposal 1: For inter-frequency / inter-RAT frequency measurement, RRM measurement relaxation for UE power saving should be:

Table 1: RRM measurement relaxation for inter-frequency / inter-RAT frequency measurement

| For inter-frequency / inter-RAT frequency measurement |  |  |  |
| --- | --- | --- | --- |
| Channel condition | Frequency | NW configures below criteria and when fulfilled | Measurement approach (or with relaxation) |
| When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Higher priority carriers | Scenarios #1: Low mobility scenario | If the UE is configured with highPriorityMeasRelax, then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. |
|  |  | Scenarios #2: Not in cell-edge scenario | the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. Note: this criterion is always fulfilled, if it is configured. |
|  |  | Scenarios #3: Low mobility and Not at cell edge | Regardless of whether the UE is configured with highPriorityMeasRelax or not, UE can stop both equal/low priority and high priority inter-frequency measurements, with a minimum measurement time interval of 1 hour since last measurement for cell reselection for higher priority carriers. |
|  | Equal/Lower priority carriers | All scenarios (#1,2,3) | UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. (According to current TS 38.304) |
| When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Higher priority carriers, and Equal/Lower priority carriers | Scenarios #1: Low mobility scenario | UE may choose to perform relaxed measurements with longer intervals. K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |
|  |  | Scenarios #2: Not in cell-edge scenario | UE may choose to perform relaxed measurements with longer intervals. K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the not-at-cell edge criterion. |
|  |  | Scenarios #3: Low mobility and Not at cell edge | UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. |

Meanwhile, for intra-frequency measurement, the following requirements are deduced from RAN2/RAN4 conclusion as Table 2.

Proposal 2: For intra-frequency frequency measurement, RRM measurement relaxation for UE power saving should be:

Table 2: RRM measurement relaxation for intra-frequency frequency measurement

| For intra-frequency measurement |  |  |  |
| --- | --- | --- | --- |
| Channel condition | Frequency | NW configures below criteria and when fulfilled | Measurement approach (or with relaxation) |
| Srxlev > SIntraSearchP and Squal > SIntraSearchQ | N/A | All scenarios (#1,2,3) | UE may choose not to perform intra-frequency measurements. (According to current TS 38.304) |
| Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ | N/A | Scenarios #1: Low mobility scenario | UE may choose to perform relaxed measurements with longer intervals. |
|  | N/A | Scenarios #2: Not in cell-edge scenario | UE may choose to perform relaxed measurements with longer intervals. |
|  | N/A | Scenarios #3: Low mobility and Not at cell edge | UE is not required to meet Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra as defined in Table 4.2.2.3-1. UE can stop intra-frequency measurement with a minimum measurement time interval of 1 hour. |

And we also proposed the draft CR [5] on requirements maintenance of measurement relaxation requirements for UE power saving. The corresponding observations should be capatured based on the proposals into TS 38.133 (including section 4.2.2.9, 4.2.2.10. 4.2.2.11).

Proposal 3: Correpondingly update the requirement for idle state measurement relaxation for UE power saving in TS 38.133 section 4.2.2.

Conclusion

In this contribution, we propose our views as follows.

Proposal 1: For inter-frequency / inter-RAT frequency measurement, RRM measurement relaxation for UE power saving should be:

Table 1: RRM measurement relaxation for inter-frequency / inter-RAT frequency measurement

| For inter-frequency / inter-RAT frequency measurement |  |  |  |
| --- | --- | --- | --- |
| Channel condition | Frequency | NW configures below criteria and when fulfilled | Measurement approach (or with relaxation) |
| When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ | Higher priority carriers | Scenarios #1: Low mobility scenario | If the UE is configured with highPriorityMeasRelax, then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. |
|  |  | Scenarios #2: Not in cell-edge scenario | the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. Note: this criterion is always fulfilled, if it is configured. |
|  |  | Scenarios #3: Low mobility and Not at cell edge | Regardless of whether the UE is configured with highPriorityMeasRelax or not, UE can stop both equal/low priority and high priority inter-frequency measurements, with a minimum measurement time interval of 1 hour since last measurement for cell reselection for higher priority carriers. |
|  | Equal/Lower priority carriers | All scenarios (#1,2,3) | UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. (According to current TS 38.304) |
| When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Higher priority carriers, and Equal/Lower priority carriers | Scenarios #1: Low mobility scenario | UE may choose to perform relaxed measurements with longer intervals. K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |
|  |  | Scenarios #2: Not in cell-edge scenario | UE may choose to perform relaxed measurements with longer intervals. K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the not-at-cell edge criterion. |
|  |  | Scenarios #3: Low mobility and Not at cell edge | UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a minimum measurement time interval of 1 hour. |

Proposal 2: For intra-frequency frequency measurement, RRM measurement relaxation for UE power saving should be:

Table 2: RRM measurement relaxation for intra-frequency frequency measurement

| For intra-frequency measurement |  |  |  |
| --- | --- | --- | --- |
| Channel condition | Frequency | NW configures below criteria and when fulfilled | Measurement approach (or with relaxation) |
| Srxlev > SIntraSearchP and Squal > SIntraSearchQ | N/A | All scenarios (#1,2,3) | UE may choose not to perform intra-frequency measurements. (According to current TS 38.304) |
| Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ | N/A | Scenarios #1: Low mobility scenario | UE may choose to perform relaxed measurements with longer intervals. |
|  | N/A | Scenarios #2: Not in cell-edge scenario | UE may choose to perform relaxed measurements with longer intervals. |
|  | N/A | Scenarios #3: Low mobility and Not at cell edge | UE is not required to meet Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra as defined in Table 4.2.2.3-1. UE can stop intra-frequency measurement with a minimum measurement time interval of 1 hour. |

Proposal 3: Correpondingly update the requirement for idle state measurement relaxation for UE power saving in TS 38.133 section 4.2.2.

References

- R4-2009265, WF on RRM measurement relaxation for Power Saving, CATT

- R4-2009133, Measurement requirements for UEs under power saving mode, Ericsson

- R4-2002338, WF on RRM measurement relaxation for NR power saving, CATT

- R4-2005330, WF on RRM measurement relaxation for NR power saving, CATT

- R4-2010705, CR on RRM Measurement relaxation requirements for UE power saving (TS 38.133), OPPO