3GPP TSG-RAN WG4 Meeting #96-e	R4-2010788

Electronic Meeting, 17th - 28th August, 2020

Source:	ZTE Corporation

Title:	Release independence support of new channel bandwidth from Rel-15

Agenda Item:	4.1

Document for:	Discussion

# Introduction

In RAN4#95-e, a WF was agreed on the support of new channel bandwidth to the existing operating bands in a manner of release independent from Rel-15 [1]. An issue was identified and confirmed for the case where a new channel bandwidth is added to an operating band introduced in Rel-15, and the new channel bandwidth is declared to be release independent from Rel-15.

And the open issue is listed as:

- How to capture in TS 38.307 Rel-15 ? - Option 1: Fill in the blank for the two tables on operating bands as in Rel-16 - Option 2: Add a sentence indicating “new channel bandwidth to an existing operating band” should be treated as a new “operating band” equivalently - Option 3: Create a new table for “UE channel bandwidth” - Option 4: Other options not precluded

# Discussion

The WF further agreed that for a new channel bandwidth added to an existing operating band in Rel-15, the release independent support from Rel-15 to the new channel bandwidth should be captured in TS 38.307 Rel-15 [1].

In Rel-16, there are 51 operating bands defined including FDD/TDD/SUL/SDL, and 36 of these 51 operating bands are introduced in Rel-15 [2][3][4][5], as shown in below table. So there is a strong need to fix the issue identified with new emerging market demands for new channel bandwidth support.

| NR operating band introduced in Rel-15 | NR operating band introduced in Rel-16 |  |
| --- | --- | --- |
| n1 | n14 |  |
| n2 | n18 |  |
| n3 | n26 |  |
| n5 | n29 |  |
| n7 | n30 |  |
| n8 | n47 |  |
| n12 | n48 |  |
| n20 | n53 |  |
| n25 | n65 |  |
| n28 | n89 |  |
| n34 | n90 |  |
| n38 | n91 |  |
| n39 | n92 |  |
| n40 | n93 |  |
| n41 | n94 |  |
| n50 | n95 |  |
| n51 |  |  |
| n66 |  |  |
| n70 |  |  |
| n71 |  |  |
| n74 |  |  |
| n75 |  |  |
| n76 |  |  |
| n77 |  |  |
| n78 |  |  |
| n79 |  |  |
| n80 |  |  |
| n81 |  |  |
| n82 |  |  |
| n83 |  |  |
| n84 |  |  |
| n86 |  |  |
| n257 | n259 |  |
| n258 |  |  |
| n260 |  |  |
| n261 |  |  |

To fix the issue, a reasonable solution is consistent with the Rel-16 version by filling the blank part of the two tables involved. Our companion draft CR can be found in [6].

Proposal : Take Option 1 to fix the issue on the release independence support from Rel-15 of a new channel bandwidth added to an existing operating band introduced in Rel-15.

# Conclusion

In this paper, we have the following proposal for the release independence support from Rel-15 of new channel bandwidth added to an existing operating band introduced in Rel-15.

Proposal : Take Option 1 to fix the issue on the release independence support from Rel-15 of a new channel bandwidth added to an existing operating band introduced in Rel-15.

# Reference

R4-2008384, WF on supporting a new channel bandwidth added to an existing operating bands in a manner of release independence from Rel-15, ZTE

TS 38.101-1 v 15.10.0

TS 38.101-1 v 16.4.0

TS 38.101-2 v 15.10.0

TS 38.101-2 v 16.4.0

R4-2010789, draft CR to TS 38.307 on release independence support from Rel-15 of a new channel bandwidth added to an existing operating band introduced in Rel-15, ZTE