3GPP TSG-WG4 Meeting #96-e	R4-2010821

# Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft correction CR for TS 38.101-3: NR V2X con-current operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some remaining issues are left to be finished for NR V2X con-current operation. Tx: switching period requirement Rx: delta Rib and MSD requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tx: specify the switching period requirement Rx: introduce MSD due to UL harmonic for V2X_20_n38. Since the delta Rib values for V2X_20_n38 are 0dB, no need to reflect 0dB in the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for NR V2X con-current opeation are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3E.1, 6.3E.2, 7.3C.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

6.3E	Output power dynamics for V2X operation in FR1

### 6.3E.1	General

The E-UTRA SL and NR SL switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For intra-band V2X operation,  the output power dynamics specified in clause 6.3.2G, 6.3.3G, 6.3.4G and 6.3.5G [4] shall be applied for E-UTRA SL transmission or the output power dynamics specified in clause 6.3E [2] shall be applied for NR SL transmission, respectively.

For the inter-band con-current NR V2X operation, output power dynamics requirement specified in subclause 6.3 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.3E of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

### 6.3E.2	Output power dynamics for intra-band V2X operation

For intra-band V2X operation bands UE specified in subclause 5.3E.1 and 5.3E.2, the maximum UL switching time masks in Figure 6.3E.2-1 and Figure 6.3E.2-2 shall apply.

Figure 6.3E.2-1: Time mask for switching between NR V2X SL and E-UTRA V2X SL

Figure 6.3E.2-2: Time mask for switching between E-UTRA V2X SL and NR V2X SL

<Next Change>

#### 7.3C.2.3	Inter-band V2X con-current operation

When UE is configured for NR V2X reception on V2X carrier con-current with E-UTRA uplink and downlink, NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.8. Also the E-UTRA downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.

For the inter-band con-current NR V2X operation, and the UE also supports a E-UTRA downlink inter-band con-current configuration in Table 5.5C.4.1-1, the minimum requirement for reference sensitivity of each band according to TS 36.101 [4] and TS 38.101-1 [2] shall be increased by the amount of ΔRIB,c for the corresponding NR V2X/E-UTRA band.

The reference sensitivity is defined to be met with Uu uplink assigned to one band (that differs from the V2X operating band) and all E-UTRA downlink carriers active. The Uu uplink resource blocks shall be located as close as possible to V2X operating band but confined within the transmission bandwidth configuration for the channel.

##### 7.3C.2.3.1	Reference sensitivity exceptions due to UL harmonic interference for V2X in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same NR V2X configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3C.2.3.1-1 with uplink configuration of the agressor band (low) specified in Table 7.3C.2.3.1-2.

Table 7.3C.2.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for V2X in NR FR1

| E-UTRA or NR V2X Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 20 | n381,2 |  | [10.3] |  | [7.4] |  |  | [4.4] |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3C.2.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for V2X in NR FR1

| E-UTRA or NR V2X Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 20 | n38 |  | 16 |  | 25 |  |  | 25 |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] applies NOTE 2: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |

## <End of Change>