3GPP TSG-RAN WG4 Meeting # 96-e                                  R4-2010885

Electronic Meeting, 17 August –28 August, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.717-21-11 DC_28A-66A_n66A

Agenda Item:	10.4.2

Document for:	Approval

Introduction

The WID for NR DC was updated in RAN #88 meeting. This contribution provides a TP for TR 37.717-21-11 to finish the UE RF requirements for the band combination.

- References

[1]	RP-201369, “New WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, HiSilicon

- Text Proposal

<TP for TR 37.717-21-11>

- 5.1.x		DC_28-66_n66

### 5.1.x.1	 Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_28-66_n66 | CA_28-66 | n66 |

### 5.1.x.2 	Configuration for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A-66A_n66A | DC_28A_n66A DC_66A_n66A2 | CA_28-66 | n66 |
| NOTE 2: Only single switched UL is supported |  |  |  |

### 5.1.x.3	 Co-existence studies

For UE coexistence study of Band 28 + Band n66, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.x.3-1.

Table 5.1.x.3-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1077 | 2413 | 2528 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 374 | 214 | 2672 | 2857 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3276 | 4123 | 4308 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 329 | 534 | 4382 | 4637 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4024 | 5833 | 6088 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2154 | 1924 | 4826 | 5056 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6417 | 6092 | 1282 | 1032 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3934 | 3634 | 1176 | 1451 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7868 | 4522 | 4772 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6836 | 5529 | 5804 |

For UL DC_66A_n66A, there is no need to study the IMD since only single switched UL is supported.

IMD4 may fall into Rx of band 66 with UL DC_28_n66.

### 5.1.x.4	 ∆TIB and ∆RIB values

Table 5.1.x.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-66_n66 | 28 | 0.6 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

Table 5.1.x.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-66_n66 | 28 | 0.2 |
|  | 66 | 0 |
|  | n66 | 0 |

### 5.1.x.5	REFSENS requirements

The reference sensitivity exception (MSD) due to IMD4 for DC_28-66_n66 with UL DC_28_n66 is specified as below referring to the MSD for DC_1A-3A_n28A from 38.101-3.

Table 5.1.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A-66A_n66A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 66 | 1729 | 5 | 25 | 2129 | 11.0 | IMD4 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |

<End of TP >