3GPP TSG-RAN WG4 Meeting # 96-e                                  R4-2010920

Electronic Meeting, 17 August –28 August, 2020

Source: 	Huawei, HiSilicon

Title: 	TP for TR 37.717-00-00 for CA_n28A-n79A_SUL_n79A-n83A

Agenda Item:	10.8.2

Document for:	Approval

- 1 Introduction

The WID for SUL band combinations was updated in RAN #88 meeting. This contribution provides a TP for CA_n28A-n79A_SUL_n79A-n83A for TR 37.717-00-00.

- References

[1]	RP-201026, “New WID on Rel-17 Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)”, RAN #88e

- Text Proposal

<TP for TR 37.717-00-00>

## 5.X	CA_n28A-n79A_SUL_n79A-n83A

### 5.X.1	Operating bands

Table 5.X.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n28-n79_SUL_n79-n83 | n28, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.X.2	Channel bandwidths per operating band

Table 5.X.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n79A_SUL_n79A-n83A | SUL_n79A-n83A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.X.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

### 5.X.5	REFSENS requirements

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.X.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n831 | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 10 | 10 |  | 10 |  |  |  |  |  |  |
| n41 | n83 | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |
|  |  | 30 |  | 10 | 10 | 10 |  | 10 |  |  |  |  |  |  |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.6	∆TIB and ∆RIB values

For CA_n28-n79_SUL_n79-n83, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n79_SUL_n79-n83 | n79 | 0.8 |
|  | n83 | 0.5 |

Table 5.X.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28-n79_SUL_n79-n83 | n28 | 0.2 |
|  | n79 | 0.5 |

<End of TP >