3GPP TSG-RAN WG4 Meeting#96-e                                 R4-2010941

E-meeting,17th Aug – 28th Aug, 2020

Agenda item:	11.1.3.2

Source: 	ZTE Corporation

Title: 	DL simulation results for 6425-7125MHz and 10-10.5GHz

Document for:	Discussion

Introduction

In the RAN4#95-e-Bis meeting, WF on simulation assumption [1] and WF for BS antenna parameters [2-3] were approved for coexistence evaluation work. Therefore in this contribution, we share some updated simulation results for further discussion.

Discussion

In the following sections, simulation results for both coordinated case and uncoordinated case in urban macro scenarios are provided based on the approved simulation assumption [1]. In addition, baseline ACIR for demonstration based on legacy FR1 NR system is assuming that using ACLR=45dBc at BS and ACS=33dBc at UE) at 7GHz and 10GHz, relative offset to baseline ACIR is used for further comparison and check whether relative relaxed requirements could be achieved.

## 2.1. Downlink

The DL simulation results for 6GHz is summarized in the following Table 1.

Table 1: DL throughput loss of victim UE at 7GHz and 10GHz

| Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| Coordinated case | Average throughput loss in % (7GHz) | 1.29 | 1.43 | 1.67 | 1.94 | 2.23 | 2.51 |
|  | 5%-tile throughput loss in % (7GHz) | 3.22 | 3.87 | 4.47 | 5.44 | 6.59 | 7.75 |
| Uncoordinated case | Average throughput loss in %(7GHz) | 1.43 | 1.59 | 1.81 | 2.06 | 2.31 | 2.60 |
|  | 5%-tile throughput loss in % (7GHz) | 4.57 | 5.17 | 5.96 | 7.01 | 8.10 | 9.43 |
| Coordinated case | Average throughput loss in % (10GHz) | 1.20 | 1.37 | 1.59 | 1.83 | 2.15 | 2.46 |
|  | 5%-tile throughput loss in % (10GHz) | 2.36 | 2.87 | 3.77 | 4.42 | 5.41 | 6.39 |
| Uncoordinated case | Average throughput loss in % (10GHz) | 1.34 | 1.53 | 1.75 | 1.98 | 2.23 | 2.50 |
|  | 5%-tile throughput loss in % (10GHz) | 4.15 | 4.71 | 5.66 | 6.96 | 8.17 | 9.62 |

Observation 1: for 7GHz, the downlink throughput loss of the victim UE in the urban macro scenario can still be limited to 5% with downlink ACIR offsets of -1 to -2dB;

Observation 2: for 10GHz, the downlink throughput loss of the victim UE in the urban macro scenario can still be limited to 5% with downlink ACIR offsets of -1 to -3dB.

Conclusions

In this contribution, we shared initial simulation results for 6.425-7.125GHz and 10.0-10.5GHz for further discussion.

Observation 1: for 7GHz, the downlink throughput loss of the victim UE in the urban macro scenario can still be limited to 5% with downlink ACIR offsets of -1 to -2dB;

Observation 2: for 10GHz, the downlink throughput loss of the victim UE in the urban macro scenario can still be limited to 5% with downlink ACIR offsets of -1 to -3dB.

References

- R4-2008927, WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz, approved.

- R4-xxx•WF R4-2008925 in BS parameters

- WF R4-2008926 in BS antenna parameters

- TR 36.942

- R4-2006108,Urban Macro Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz, Noted