3GPP TSG-RAN WG4 Meeting #96-e	R4-2010980

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Discussion on URLLC CSI requirements with ultra-low BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.1.2

Document for:	Discussion

Background

In this paper, the CQI reporting test with ultra-low BLER is discussed. According to the WF [1], some agreements were made during the GTW session and some open issues were remained to be discussed further:

Open issues:

- CQI requirement

  - Option 1: No CQI requirement

  - Option 2: CQI requirement with median CQI and BLER with higher/lower CQI followed, together with test applicability rule on CQI/FMCS

  - Recommended WF

-> Keep it open, further check test feasibility for CQI reporting and make decision in Q3 2020.

-> If RAN4 can concluded test time is comparable to FMCS test cases, then RAN4 decide to introduce static CQI test case with ultra-low BLER

-> FFS whether we need to introduce test applicable rules among CQI/FMCS test cases if CQI test cases introduced

-> Companies also encourage to bring analysis whether static CQI/fading CQI test with non-ultra-BLER can serve test purpose

- Test applicability rule for FMCS and CQI

  - Only applicable if CQI test is introduced

  - Option 1: Introduce applicability rule

  - Option 2: No rule; both tested

In this paper, the open issues are discussed and our views are provided.

Discussion

- Test applicable rule for FMCS and CQI:

As we discussed in previous papers, the CQI test criteria under fading channel for ultra-low BLER test is not feasible. So we propose to define the CQI test for ultra-low BLER under AWGN channel and reuse the BLER criteria.

We think it is not feasible to define CQI reporting test case and FMCS case at the same SNR for all possible situation:

For example, in situation:

- If UE shows < 1e-5 BLER at the same SNR for an MCS greater than or equal to MCS in fixed MCS test, UE automatically passes the fixed MCS test.

But the MCS is smaller than the MCS in FMCS test. Or in situation:

- If UE shows > 1e-5 BLER at the same SNR for an MCS less than or equal to MCS in fixed MCS test, UE automatically fails the fixed MCS test.

But the MCS is greater than the MCS in FMCS test.

It cannot always reduce the test efforts in any scenario, but it will complex the test procedure and maybe increase the test efforts.

Proposal 1: We propose to define the CQI test for ultra-low BLER under AWGN channel and reuse the BLER criteria.

Proposal 2: No rule; both of FMCS and CQI tested if CQI reporting test is agreed to be introduced in RAN4.

Proposals

In this paper, following are proposed:

Proposal 1: We propose to define the CQI test for ultra-low BLER under AWGN channel and reuse the BLER criteria.

Proposal 2: No rule; both of FMCS and CQI tested if CQI reporting test is agreed to be introduced in RAN4.

Reference

[1] R4-2008805, Way forward on ultra-low BLER requirements, #95, Ericsson.