3GPP TSG-RAN WG4 Meeting #96-e	R4-2010982

# Electronic Meeting, 17th - 28th Aug, 2020

Title: 	Simulation results on URLLC UE demodulation requirements with higher BLER

Source: 	Huawei, HiSilicon

Agenda item:	7.8.1.2.1

Document for:	Discussion

Background

In this paper, the simulation results for URLLC UE demodulation requirements are presented based on the agreed parameters on previous e-meetings.

High reliability

Simulation assumption

For PDSCH high reliability with higher BLER, MCS13/16/19 are proposed by companies. The simulation results for these three MCS are shown in this paper. The agreed parameters in WF [1] are concluded in table below.

Table 2-1: Simulation assumption for UE PDSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Duplex mode |  | FDD/TDD |
| Antenna configuration |  | 2x2, ULA low 2x4, ULA low |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ Processes |  | 4 |
| PT-RS |  | Disabled |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 3, MCS 13/16/19 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| Frequency domain resource |  | Full BW |
| TDD pattern |  | 7D1S2U (S=6:4:4) |
| Testing metric |  | Target BLER: 1% (BLER is calculated after all transmission) |

Simulation results:

PDSCH performance with different MCS for TDD and FDD were simulated.

- FDD

The SNR values with MCS13/16/19 when BLER=0.01 for 2x2 and 2x4 are shown in table below:

Table 2-2: SNR values for PDSCH FDD performance requirements

| MCS Antenna configuration SNR | 2x2 | 2x4 |
| --- | --- | --- |
| MCS13 | -1.6 | -4.4 |
| MCS16 | 1.7 | -1.3 |
| MCS19 | 5 | 2.1 |

Low latency

PDSCH mapping Type B and processing capability 2:

Simulation assumption

The test parameters according to the discussion of previous meetings are concluded in table below:

Table 3-1: Test parameters for UE PDSCH performance requirements

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Duplex mode |  | FDD/TDD |
| Antenna configuration |  | 2x2 and 2x4, ULA low |
| PDSCH configuration | Mapping type | Type B |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 2 |
|  | PDSCH aggregation factor | 1 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Maximum number of HARQ transmission |  | 1 |
| Number of HARQ process |  | 2 |
| Propagation condition |  | TDLA30-10 |
| MCS Table |  | Table 1, MCS 4 |
| SCS and BW |  | FDD:15KHz, 10MHz TDD:30KHz, 40MHz |
| TDD Pattern |  | DDDSU, S=10:2:2 |
| Frequency domain resource |  | Full BW |
| K1 |  | 0 |
| Testing metric |  | Throughput: 70% |

Simulation results:

PDSCH performance with for FDD and TDD were simulated based on the parameter assumption in Table 3-1.

- FDD

The performance for 2x2 and 2x4 for symbol lengths of 2 and 7 are simulated as shown in figure below (L=2os has been agreed):

Figure 3-1: Throughput vs SNR for PDSCH FDD with symbol lengths of 2 and 7 for antenna configuration 2x2 and 2x4

According to the simulation results shown in Figure 2-1, the SNR values of 70% throughput are listed in table below:

Table 3-2: SNR values of 70% throughput for FDD

| Symbol length SNR Antenna configuration | 2x2 | 2x4 |
| --- | --- | --- |
| L=2 | -2.4 | -5.7 |

- TDD

The PDSCH mapping Type B for TDD was agreed that only scheduled on S slot. The simulation results of PDSCH TDD with symbol lengths of 2 and 7 are shown in below (L=2os has been agreed):

Figure 3-2: Throughput vs SNR for PDSCH TDD with symbol lengths of 2 and 7 for antenna configuration 2x2 and 2x4

According to the simulation results shown in Figure 3-2, the SNR values of 70% throughput are listed in table below:

Table 3-3: SNR values of 70% throughput for TDD

| Symbol length SNR Antenna configuration | 2x2 | 2x4 |
| --- | --- | --- |
| L=2 | -2.2 | -5.5 |

Pre-emption indication:

Simulation assumption

The agreed parameters are concluded in table below.

Table 3-3: Simulation assumption for UE PDSCH performance requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD, TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | FDD: 4 TDD: 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information and TDD pattern |  |  | FDD: 2 TDD: FR1.30-1 |
| CBW/SCS |  |  | FDD: 10/15 TDD: 40/30 |
| RB allocation |  |  | Full bandwidth |
| MIMO layer |  |  | Rank 1 |
| Propagation condition |  |  | TDLA30-10 |
| Test metric |  |  | 70% TP |

Simulation results

According to the agreed parameters for URLLC and parameters in table above for eMBB, simulations were setup for FDD with MCS4 and MCS13. The results are shown in figure below, simulations include 1). Throughput vs BLER for eMBB UE without pre-emption. 2). Throughput vs BLER for eMBB UE with pre-emption of 10% probability with/without buffer flushing. 3). Throughput vs BLER for eMBB UE with pre-emption of 20% probability with/without buffer flushing.

Figure 3-3: PDSCH performance for pre-emption with MCS4 and MCS13

The SNR values of these setup for 70% of maximum throughput are shown in table below:

Table 3-4: SNR values for PDSCH FDD performance requirements

| Antenna configuration SNR Setup |  | MCS4 | MCS13 |
| --- | --- | --- | --- |
| No pre-emption |  | -3.6 | 4.8 |
| With buffer flush | 10% probability | -3.5 | 6.1 |
|  | 20% probability | -3.2 | 8.2 |
| Without buffer flush | 10% probability | -3.3 | 7 |
|  | 20% probability | -3 | 13 |

Reference

- R4-2008807, “Way forward for NR UE URLLC performance requirements”, Intel Corporation.