3GPP TSG-RAN WG4 Meeting #96-e	R4-2011026

# Electronic Meeting, 17-28 Aug., 2020

Title: 	Discussion on NR CA CQI reporting requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.16.1.5

Document for:	Discussion

Background

In RAN 4 95-e meeting, the WF [1]   was approved. In this paper, we will give our discussion on remaining details about NR CA

CQI reporting requirements

Discussion

Duplex mode and SCS combinations

The options are shown as follows:

- Duplex mode and SCS combinations - For the performance requirements: - Option 1: Reuse the combinations from PDSCH normal - Option 2: - FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS - FR2: TDD + TDD with 120 kHz SCS - Applicability rule if option 1 for the performance requirements is agreed: - Option 1: Test 2 of the 3 cases below, and FFS on the detailed applicability rule: - Test #1: FDD 15 kHz + FDD 15 kHz - Test #2: FDD 15 kHz + TDD 30 kHz, in case UE supports different SCS on different carriers for FDD-TDD CA, otherwise FDD 15 kHz + TDD 15 kHz - Test #3: TDD 30 kHz + TDD 30 kHz, in case UE supports it, otherwise TDD 15 kHz + TDD 30 kHz - Other options are not precluded

We prefer option 2. Supporting of different CA combinations are covered in PDSCH CA performance requirements and no need to repeat to verify them in CA CQI test. FDD CA and TDD CA with same numerology is mandatory for UE to support, it is enough to check the CA CQI performance by using the CA with same numerology.

Proposal 1: Use Duplex mode and SCS combinations as following:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

Channel bandwidth and test applicability rule

Based on our observations, it is feasible to define the CA CQI performance requirements in bandwidth agnostic way. For test applicability rule, we prefer to reuse the LTE approach.

Proposal 2: Define the CA CQI performance requirements in bandwidth way and reuse the test applicability rule of LTE approach.

TDD pattern for 120kHz SCS

The options are shown as follows:

- TDD pattern for 120kHz SCS - Option 1: 3D1S1U with S=10:2:2 - Option 2: 2D1S1U with S=11:3:0

We support option 1 of DDDSU since it is the most typical pattern. Also the UL PUSCH performance requirements only consider this TDD pattern for 120 kHz SCS.

Proposal 3: Use DDDSU as TDD pattern for 120 kHz SCS

General principle

The options are shown as follows:

- General principle - Following the methodology used in LTE, the difference between the wideband CQI indices of Pcell and the first Scell as well as the difference between the wideband CQI indices of the first Scell and the other Scell(s) (if any) shall be not smaller than a threshold, for more than 90% of the time - SNR configuration for 2DL CA CQI test - Option 1: SNRPcell = 10dB and SNRScell = 4dB - Other options are not precluded - Make decision in the next meeting - SNR configuration for 3 or more DL CA CQI test - Option 1: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB - Other options are not precluded - Make decision in the next meeting - Delta CQI threshold for CA CQI test - Option 1: thr = 2 for 2 or more DL CA - Other options are not precluded - Make decision in the next meeting

Table 1 and Table 2 are simulation results. The SNR configuration is used from option 1.

Table 1: Distribution of delta CQI for 2 DL CA (SNRPcell = 10dB, SNRScell = 4dB, PCell: 10MHz, PCell: 10MHz, SCell: 10MHz)

| Test metric | wideband CQIPcell – wideband CQIScell ≥ x |  |  |
| --- | --- | --- | --- |
| Percentage | 1 | 2 | 3 |
|  | 100% | 98.4% | 75.8% |

Table 2: Distribution of delta CQI for 3 DL CA (SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2 = 0dB, PCell: 10MHz, SCell1: 10MHz, SCell2: 10MHz)

| Test metric | wideband CQIPcell – wideband CQIScell1 ≥ y |  |  |
| --- | --- | --- | --- |
| Percentage | 1 | 2 | 3 |
|  | 100% | 97.7% | 72.2% |
|  | wideband CQIScell1 – wideband CQIScell2 ≥ z |  |  |
|  | 1 | 2 | 3 |
|  | 100% | 96.3% | 60.4% |

From our simulation results, we can observe that:

Observation 1:  For 2 DL CA and SNRPcell = 10dB, SNRScell = 4dB, the difference between the wideband CQI indices of the first Pcell and the Scell IS lager than 2 for 98.4% of the time and lager than 3 for 75.8% of the time

Observation 2:  For 3 DL CA and SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2 = 0dB, the difference between the wideband CQI indices of the first Pcell and the Scell1 is lager than 2 for 97.7% of the time and lager than 3 for 72.2% of the time. The difference between the wideband CQI indices of the first Scell1 and the Scell2 is lager than 2 for 96.3% of the time and lager than 3 for 60.4% of the time.

From the observation, we can reuse the general principle, SNR configuration and delta CQI threshold from LTE.

Proposal 4: General principle

  - Following the methodology used in LTE, the difference between the wideband CQI indices of Pcell and the first Scell as well as the difference between the wideband CQI indices of the first Scell and the other Scell(s) (if any) shall be not smaller than a threshold, for more than 90% of the time

SNR configuration for 2DL CA CQI test

  - SNRPcell = 10dB and SNRScell = 4dB

SNR configuration for 3 or more DL CA CQI test

  - Option 1: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB

Delta CQI threshold for CA CQI test

  - thr = 2

Conclusion

In this paper, we will give our discussion on remaining details about NR CA CQI reporting requirements. Our proposals are:

Proposal 1: Use Duplex mode and SCS combinations as following:

- FR1: FDD + FDD with 15 kHz SCS and TDD + TDD with 30 kHz SCS

- FR2: TDD + TDD with 120 kHz SCS

Proposal 2: Define the CA CQI performance requirements in bandwidth way and reuse the test applicability rule of LTE approach.

Proposal 3: Use DDDSU as TDD pattern for 120 kHz SCS

Proposal 4: General principle

  - Following the methodology used in LTE, the difference between the wideband CQI indices of Pcell and the first Scell as well as the difference between the wideband CQI indices of the first Scell and the other Scell(s) (if any) shall be not smaller than a threshold, for more than 90% of the time

SNR configuration for 2DL CA CQI test

  - SNRPcell = 10dB and SNRScell = 4dB

SNR configuration for 3 or more DL CA CQI test

  - Option 1: SNRPcell = 12dB, SNRScell1 = 6dB, SNRScell2, 3,… = 0dB

Delta CQI threshold for CA CQI test

  - thr = 2

Reference

- R4-2008848 Way forward on UE power imbalance requirements for FR1 CA and EN-DC. NTT DOCOMO

- R4-2006039 FR1 CA PDSCH demodulation requirement with power imbalance. China Telecom.