3GPP TSG-RAN WG4 Meeting #96-e	R4-2011041

Electronic Meeting, 17-28 Aug., 2020

Agenda item:		7.10.3.1

Source:				NTT DOCOMO, INC.

Title:		Views on 256QAM UE requirements for FR2

Document for:	Discussion

1. Introduction

At the last RAN4 meeting, we discussed 256QAM UE requirements and RAN4 agreed a WF [1]. This contribution presents our views on 256QAM UE requirements.

2. Discussion

  - Simulation results

Table 1 shows our simulation results based on the simulation parameters [1].

Observation 1

Table 1. Summary of ideal simulation results

| Rank | Channel Model | Antenna configuration | CBW (MHz) | SNR point (dB) @70%TP Using MCS index: |
| --- | --- | --- | --- | --- |
|  |  |  |  | 20 |
| 1 | TDLD 30-35 | 2x2 | 50 | 16.5 dB |
| 2 | Static | 2x2 | 50 | 19.9 dB |

Observation 2: Considering around 3dB impairment margin, TDL-D 30-35 is testable under 50MHz CBW with full PRB allocation.

  - Rank, Propagation condition

Rank configuration

Based on our simulation results, rank 2 for FR2 256QAM is quite severe condition since maximum SNR for FR2 test is quite limited.

Observation 3: Define DL 256QAM demodulation performance requirements for rank 1

Propagation condition

Propagation condition for FR2 256QAM was also discussed at the last meeting, and RAN4 has not reached the consensus. The possible options are TDL-A 30-300, TDL-D 30-75, TDL-D 30-35 and Static channel [1].  Considering that performance gain from DL 256QAM would be quite limited in NLOS condition, it is natural to assume LOS propagation model, i.e. TDL-D, for DL 256QAM. On the other hand, only for Rank 1, some performance gain would be considered even in NLOS condition. Therefore, our preference is still TDL-A and TDL-D However, we are fine to discuss further based on simulation results.

Proposal 1: Consider following options for propagation condition for FR2 DL 256QAM

- For Rank 1

  - TDL-D (30-35 or 30-75) or TDL-A (30-300)

3. Conclusion

In this contribution, we present our views on 256QAM UE requirements for FR2. Our proposals and observations are summarized below.

Observation 1

Table 1. Summary of ideal simulation results

| Rank | Channel Model | Antenna configuration | CBW (MHz) | SNR point (dB) @70%TP Using MCS index: |
| --- | --- | --- | --- | --- |
|  |  |  |  | 20 |
| 1 | TDLD 30-35 | 2x2 | 50 | 16.5 dB |
| 2 | Static | 2x2 | 50 | 19.9 dB |

Observation 2: Considering around 3dB impairment margin, TDL-D 30-35 is testable under 50MHz CBW with full PRB allocation.

Proposal 1: Consider following options for propagation condition for FR2 DL 256QAM

- For Rank 1

  - TDL-D (30-35 or 30-75) or TDL-A (30-300)

References

- R4-2008817, China Telecom, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, RAN4 #95e