3GPP TSG-RAN WG4 Meeting # 96-e	R4-2011186

# Electronic Meeting, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0068 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.817-02: Clarification on calculation of step frequencies for defining the Category B radiated Tx spurious emission limits in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The step frequencies for defining the Category B radiated Tx spurious emission limits in FR2 are calculated based on ERC Recommendation 74-01 and ΔfOBUE, but how to calculate them are not clearly explained in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clearly explain how to calculate the step frequencies for defining the Category B radiated Tx spurious emission limits in FR2 for future reference when the limits are applicable to other bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains and would lead to misinterpretation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 9.7.5.3	AAS radiated Tx spurious emissions requirement for FR2

The Tx spurious emission requirement for BS type 2-O is defined at RIB for the spurious range from 30 MHz up to the 2nd harmonics of the upper frequency edge of the operating band.

Category A limits are defined for FR2 and are the same as limits defined for basic limits in subclause 6.6.5.2. The limits are specified in table 9.7.5.3.2-1 of TS 38.104 [3].

Category B limits are defined for FR2 with regional regulation defined in [35] as baseline. The limits are specified in table 9.7.5.3.2.3-1 with step frequencies defined in table 9.7.5.3.2.3-2. Currently, step frequencies are only defined for one band, i.e. n258, for Category B spurious emissions. If the applicability of Category B limit is confirmed for other FR2 operating band in future, corresponding step frequencies for that band will be defined accordingly. The step frequencies are calculated as shown in Table 9.7.5.3-0.

Table 9.7.5.3-0: (Void)

| Operating band (GHz) | Fstep,1 (GHz) | Fstep,2 (GHz) | Fstep,3 (GHz) (Note 2) | Fstep,4 (GHz) (Note 2) | Fstep,5 (GHz) | Fstep,6 (GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| n258 (24.25 – 27.5) | max (18, 24.25 - max (10, 4 × WB)) = 18 | 24.25 - max (2, WB) = 21 | 24.25 - 1.5 = 22.75 | 27.5 + 1.5. = 29 | 27.5 + max (2, WB) = 30.75 | 27.5 + max (10, 4 × WB) = 40.5 |
| NOTE 1: max () is the maximum operation of the two operands. NOTE 2: WB is the bandwidth of the operating band. |  |  |  |  |  |  |

For conformance testing of the Tx spurious emissions requirement for BS type 2-O, the upper frequency limit of the spurious range might be limited to 60 GHz value, considering practical OTA measurement capabilities of the OTA test ranges in the spurious range.

Table 9.7.5.3-1: (Void)

<End of change>