3GPP TSG-RAN WG4 Meeting # 96-e	R4-2011216

# Electronic Meeting, 17 August - 28 August 2020

Agenda item:	12.1.3

Source:	Keysight Technologies

Title:	On FR2 NR MIMO OTA Testing Methodology

Document for:	Approval

# Introduction

This contribution is addressing open topics from the SI and items that were missed and need further clarifications in the WI.

# Discussion

While the NR MIMO OTA SI [1] was concluded, a NR MIMO WI [2] was opened in RAN#88-e. The last RAN4 meeting of the SI defined revised probe locations [3][4]as well as definition of the device rotation in Clause A.2

In order to achieve the FR2 test points tabulated in Table 6.2.3.2-1, the UE is rotated so that the test point w.r.t. to the UE coordinate system is aligned with the test system z axis.

The WF from that meeting [5] captured as next step the need to visualize the DUT rotations

- Potential work in WI phase - Further work is suggested to illustrate the DUT rotations.

The reason for potential confusion of the DUT rotations for this NR FR2 MIMO OTA test methodology is based on the system utilizing 6 probes around the y axis while the reference axis for DUT rotations is the z axis where OTA systems commonly have a measurement probe.

The 36 test points/coordinates are defined in [6], specifically Table 6.2.3.2-1.

Table 6.2.3.2-1. Evenly spaced FR2 test points with a constant density Test Point Number; Theta [deg]; Phi [deg]. 1; 0.0; 0.0. 2; 33.5; 139.7. 3; 33.9; 49.7. 4; 35.5; -142.9. 5; 35.5; -76.9. 6; 37.6; -17.2. 7; 52.3; 94.7. 8; 56.9; 175.7. 9; 62.5; 20.4. 10; 63.7; -99.8. 11; 67.1; -55.0. 12; 69.3; -139.5. 13; 69.5; 130.1. 14; 70.3; 60.8. 15; 72.1; -16.2. 16; 88.7; -167.5. 17; 88.7; 98.5. 18; 89.3; 157.0. 19; 93.9; -78.9. 20; 94.6; 31.6. 21; 95.3; -115.6. 22; 99.6; -38.3. 23; 103.8; -1.1. 24; 104.4; 66.3. 25; 110.1; 127.5. 26; 115.1; -145.6. 27; 120.8; 171.9. 28; 125.3; -60.7. 29; 128.2; -104.1. 30; 128.8; 91.3. 31; 129.9; 35.8. 32; 136.0; -13.4. 33; 145.8; 138.1. 34; 150.2; -153.3. 35; 160.6; -67.4. 36; 161.7; 59.1.

To visualize the DUT rotations, two simple test points, not part of the test points of Table 6.2.3.2-1, are considered first. One sample test point, i.e., point #2 from Table 6.2.3.2-1, is visualized as well as the resulting DUT rotation.

Those rotations are visualized in Table 1 for an example NR FR2 MIMO OTA MPAC system utilizing a roll-over-AZ combined-axes positioning system. The 6 NR MIMO probes clustered around the y axis are visualized with small blue spheres which can be implemented using small mm-wave probes or IFF reflectors.

The second column visualizes the selected test points (q,f) with respect to the DUT with a black sphere. The first two test points are chosen for simplicity as follows:

- (90o, 0o) along the x test system axis is the direction pointing outside the screen

- (90o, 90o) along the y test system axis is the direction pointing to the right of the DUT

The DUT rotations for these test points are shown in the third column of Table 1. For each case, the relative orientation of the black sphere (which is now aligned with the z axis) with the DUT has been maintained, e.g., the direction perpendicular to the screen is aligned with the test system z axis for example test point (90o,0o) and the direction to the right of the screen is aligned with the test system z axis for example test point (90o,90o). The last row illustrates the second test point in Table 6.2.3.2-1 as well as the required DUT rotation.

Table 1: Visualization of DUT Rotations for a sample NR FR2 MIMO MPAC system

| Test Point (q/f) [o] | Illustration of Test Point | Illustration of DUT Rotation |
| --- | --- | --- |
| 90,0 |  |  |
| 90,90 |  |  |
| #2 of Table 6.2.3.2-1 (33.5, 139.7) |  |  |

It is proposed to include this example illustration in Table 1 in the new WI TS or in the SI TR [6].

Proposal 1: Include this example illustration in Table 1 in the new WI TS or in the SI TR [6]

As outlined in [3], the example positioner and probe configuration shown in this contribution does not introduce any blocking effects as the probes are placed where the positioner mast does not obstruct the path between the UE and the NR MIMO probes. The test cases can therefore be performed without the re-positioning approach and a more limited QoQZ validation approach for these configurations without blocking should therefore be defined in the WI phase.

Proposal 2: For positioner/probe configurations that do not introduce blocking, the new WI TS shall capture that a limited QoQZ validation approach (similar to re-positioning concept) is sufficient.

For positioner/probe configurations that introduce blocking, the re-positioning concept can be applied to the test case and QoQZ validation to avoid the blocking effect and reduce the QoQZ MU. This approach has not been properly documented in [6] yet and will need to be captured in the WI TS. It should furthermore be noted that the QoQZ validation approach in [7][8] is assuming the measurement probes in the z direction, while the NR MIMO probes are in the y direction. It is proposed to further clarify the re-positioning concept with respect to the QoQZ validation for NR FR2 MIMO in the WI.

Proposal 3: For positioner/probe configurations that introduce blocking, the new WI TS shall capture that the re-positioning concept can be applied to the test cases and QoQZ validation.

Proposal 4: The re-positioning concept with respect to the QoQZ validation needs to be further clarified in the WI

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Include this example illustration in Table 1 in the new WI TS or in the SI TR [6]

Proposal 2: For positioner/probe configurations that do not introduce blocking, the new WI TS shall capture that a limited QoQZ validation approach (similar to re-positioning concept) is sufficient.

Proposal 3: For positioner/probe configurations that introduce blocking, the new WI TS shall capture that the re-positioning concept can be applied to the test cases and QoQZ validation.

Proposal 4: The re-positioning concept with respect to the QoQZ validation needs to be further clarified in the WI

# References

- RP-181402, New SID on Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs, CATR, OPPO, Samsung, 3GPP TSG-RAN Meeting #80, June 2018

- RP-201302, “New WID on Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements for NR UEs,” CAICT, OPPO, 3GPP RAN#88-e, June 2020

- R4-2006743, On FR2 3D MPAC Ambiguities and Blocking, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #95-e, June 2020

- R4-2008273, TP to TR38.827 to avoid ambiguities for FR2 MIMO OTA Testing, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #95-e, June 2020

- R4-2008865, WF on FR2 MIMO OTA, CAICT, Spirent, Keysight, 3GPP TSG-RAN WG4 Meeting #95-e, June 2020

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE), V2.0.0 (2020-06)

- TR38.810, Study on test methods, V16.5.0 (2019-12)

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.40 (2020-06)