3GPP TSG-RAN4 Meeting #96-e		R4-2011240

# Electronic Meeting, August 17-28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BWP switching delay requirement due to consistent UL failure in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To transmit timing requirements in NR-U |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is based on the draft CR in R4-2008571 endorsed in RAN4#95-e. The following updates are done related to the two editor’s notes: The UE capability of, “ul-LBT-FailureDetectionRecovery” is used and is referred to RRC spec. The delay due to availability of RACH resource in the BWP switching delay requirement is added to make it consistent with other requirements where UE sends RACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BWP switch delay under consistent UL LBT failure in SpCell cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.1, 8.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

## 8.6	Active BWP switch delay

### 8.6.1	Introduction

The requirements in this clause apply for a UE configured with more than one BWP on PCell or any activated SCell in standalone NR or NE-DC, PCell, PSCell or any activated SCell in MCG or SCG in NR-DC, or PSCell or any activated SCell in SCG in EN-DC. UE shall complete the switch of active DL and/or UL BWP within the delay defined in this clause. The requirements in this section also apply for a UE configured with more than one BWP on PCell or any activated SCell with CCA in standalone NR, or PSCell or any activated SCell with CCA in SCG in EN-DC. The requirements in 8.6.4 apply for a UE, which is capable of ul-LBT-FailureDetectionRecovery [14] and configured with more than one UL BWP on PCell with CCA in standalone NR or PSCell with CCA in EN-DC.

----------------------NEXT CHANGE----------------------------

### 8.6.4	BWP switch delay for consistent UL LBT failure and recovery

Upon detection of consistent UL LBT failure is slot#n in SpCell when UE detects lbt-FailureInstanceMaxCount number of LBT failure within lbt-FailureDetectionTimer, the UE shall switch the active UL BWP to an UL BWP configured with PRACH occasion and for which consistent LBT failure has not been triggered as defined in TS 38.321 clause 5.21 [7]. The UE shall be able to transmit PRACH on the new UL BWP of the SpCell on the first UL slot occurs right after slot n+TBWPswitchDelay+1+TIUTslot length. The UE shall finish the UL BWP switch within the time duration TBWPswitchDelay depending on UE capability bwp-SwitchingDelay [2].

Where:

- TBWPswitchDelay is defined in Table 8.6.2-1.

- TIU is the interruption uncertainty due to the random access procedure when sending PRACH on the new UL BWP of the SpCell. TIU can be up to: (1+L)TSSB,RO + 10 ms; where TSSB,RO is the SSB to PRACH occasion association period defined in the table 8.1-1 of TS 38.213 [3] and L is the number of consecutive number of SSB to PRACH occasion associated periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure. L=0 for Type 2C UL channel access procedure as defined in TS 37.213 [33].

- Tslot length is the NR slot length expressed in ms.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay on the SpCell in the UL BWP switch. The UE is not required to follow the requirements defined in this clause when performing a UL BWP switch between the UL BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

----------------------END OF CHANGES----------------------------