3GPP TSG-RAN WG4 Meeting #96e		R4-2011374

Electronic Meeting, August 17th – August 28th, 2020

Agenda item:	7.10.3.1

Source:	Ericsson

Title:	UE demodulation requirements for FR2 DL 256QAM​

Document for:	Discussion

# Introduction

In the WF from RAN4#95-e [1], some agreements were reached for UE demodulation requirement tests for FR2 DL 256QAM, while some open issues still remain. In this paper we provide with our simulation results, and views on the simulation assumptions for these tests.

# Discussion

## Simulation assumptions

The simulations are based on the following simulation assumptions.

- Tx EVM​: 3% ​

- Rank​: Rank 1

- Channel bandwidth and PRB allocation​

  - Option 1: 100MHz CBW with full PRB allocation​

  - Option 2: 50MHz CBW with full PRB allocation​

- Propagation condition​

  - Option 1: Fading channel​

  - Option 1A: TDLA30-300 ​

  - Option 1B: TDLD30-75​

  - Option 1C: TDLD30-35 ​

  - Option 2: Static channel​

- MCS​

  - MCS 20 for rank 1

- MIMO configuration​

  - Option 1: 2Tx 2Rx ULA low​ for fading channel

  - Option 2: 1Tx 2Rx​ for static channel

- DM-RS configuration​ : Type 1 single symbol front loaded, 1 additional DMRS​

- PRB bundling size: 2​

- Precoding model: Random Precoding, per slot , WB granularity​

- HARQ process number​: 8​

- SCS: 120kHz

- PT-RS configuration​: per 2PRB in frequency domain, per symbol in time domain​

- Overhead for TBS determination: 6

- PDSCH mapping type​: Type A

- TDD configuration: DDDSU, S=10D:2G:2U​

- PDCCH duration​: Symbol #0 in each slot​

- PDSCH configuration​:

  - Full DL slots: Start symbol 1, Duration 13

  - Special slots: Start symbol 1, Duration 9

- SSB configuration: Periodicity 20 ms, allocated in first slot within 20ms​

- TRS configuration​: 20 ms periodicity, 2 slots, Offset 10 ms​

- HARQ RV sequence: {0, 2, 3, 1}​

- Receiver assumption: MMSE-IRC

- Test metric: 70% of max TP

## Simulation results and proposals

Simulation results for TDLA30 and AWGN channel conditions, and for 50MHz and 100MHz CBW, are given below based on the above simulation assumptions.

Figure 1 Throughput curve for 50MHz CBC

Figure 2 Throughput curve for 100MHz CBC

Table 1 SNR at 70% Throughput target

| CBC [MHz] | Channel Model | SNR @ 70% throughput mark [dB] |
| --- | --- | --- |
| 50 | TDLA30-300 | 17.3 |
|  | AWGN | 15.3 |
| 100 | TDLA30-300 | 17.2 |
|  | AWGN | 15.2 |

We do not think testing demodulation requirements with static channel models should be considered. There are already defined REFSENS scenarios with static channel models in place.

Observation 1: Under ideal simulations (without impairment), the 70% testing point is close to the SNR limit with OTA testing.

Proposal 1: Do not create demodulation requirements using static channel model.

# Conclusion

In summary, we have performed UE demodulation simulations for FR2 DL 256QAM in accordance with simulation assumptions found in WF [1]. We have the following observations and proposals:

Observation 1: Under ideal simulations (without impairment), the 70% testing point is close to the SNR limit with OTA testing.

Proposal 1: Do not create demodulation requirements using static channel model.

# References

- R4-2008817, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom