3GPP TSG-RAN4 Meeting #97-e	R4-2014015

# Online, , 2nd Nov 2020 - 13th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0079 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Noc for NR operating bands in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Rel-15 FR2 multi-band requirement framework was updated in R4-2006352, and introduces a maximum cap to the per-band relaxation factors. Clause 4.5.3 needs to be aligned to these changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update formulae and Table notes to use per-band relaxation factors ∆MBP,n (as demodulation and CSI requirements are specified in Rx Beam peak direction, ∆MBS,n is not relevant). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Noc levels would be misaligned with core requirements, and tests would not be carried out in the intended condition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 4.5.3	Noc

#### 4.5.3.1	Introduction

For Mode 1 conditions radiated testing of demodulation and CSI requirements it is not feasible in practice to use signal levels high enough to make the noise contribution of the UE negligible. Demodulation requirements are therefore specified with the applied noise higher than the UE peak EIS level in TS 38.101-2 [7] by a defined amount, so that the impact of UE noise floor is limited to no greater than a value ∆BB at the specified Noc level. As UEs have EIS levels that are dependent on operating band and power class, Noc level is dependent on operating band and power class.

#### 4.5.3.2	Noc for NR operating bands in FR2

Values for Noc according to operating band and power class for single carrier requirements are specified in Table 4.5.3.2-1 for ∆BB =1dB.

Table 4.5.3.2-1: Noc power level for different UE power classes and frequency bands

| Operating band | UE Power class |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| n257 | -166.8 | -161.3 | -157.6 | -166.3 |
| n258 | -166.8 | -161.3 | -157.6 | -166.3 |
| n260 | -163.8 |  | -155.0 | -164.3 |
| n261 | -166.8 | -161.3 | -157.6 | -166.3 |
| Note 1: Noc levels are specified in dBm/Hz |  |  |  |  |

For PC3 multi-band devices, the Noc power level (NocMB) shall increase by multi-band relaxation defined in Table 6.2.1.3-4 of TS 38.101-2 [7]:

NocMB = NocSB + ∆MBP,n

-	NocSB is the Noc defined in Table 4.5.3.2-1

-	∆MBP,n values are specified in TS 38.101-2 [7].

For CA case, the Noc power level (NocCA) shall increase by a relaxation factor defined in TS 38.101-2 [7] Table 7.3A.2.1-1:

NocCA = NocSC + ΔRIB

-	NocSC is derived by assuming UE supports single carrier.

-	ΔRIB values are specified in TS 38.101-2 [7].

<< End of changes >>