3GPP TSG-RAN WG4 Meeting #97-e                                                          R4-2014036

Electronic Meeting, 2 - 13 November 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717.21-11: Addition of DC configuration for DC_20-32_n3

Agenda item:	10.4.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_20A-32A_n3A as defined in WID on Dual Connectivity of 2 band LTE (2DL/1UL) and 1 NR band (1DL/1UL) [1].

# Text Proposal

##### ---Start of changes---

## 5.x	DC_20-32_n3

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-32A_n3A | DC_20A_n3A |

### 5.x.2	Co-existence studies

Co-existence analysis from DC_20_n3 shows that no IMD impact on the DL of band 32 from the dual UL.

### 5.x.3	∆TIB and ∆RIB values

For DC_20-32_n3, the TIB,c and RIB,c values are reused from the LTE combination CA_3-20-32, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-32-n3 | 20 | 0.3 |
|  | n3 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-32-n3 | 20 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

### 5.x.4	REFSENS requirements

No additional MSD requirement is needed.

##### ---End of changes---

# Reference

[1]	RP-201728, “WID revision Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, RAN#89-e