3GPP TSG-RAN WG4 Meeting #97-e                                                         R4-2014103

Online, 2nd-13th Nov, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.717-21-11: DC_1-3_n3

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_1-3_n3 according to the request in [1].

# 2. Reference

- RP-201728, Revised WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_1-3_n3

### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n3A | DC_1A_n3A DC_3A_n3A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.x.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And there is no IMD issue for this DC configuration.

### 5.x.3	∆TIB and ∆RIB values

For DC_1-3_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from DC_1_n3.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3 | 1 | 0 |
|  | 3 | 0 |
|  | n3 | 0 |

### 5.x.4	REFSENS requirements

No additional MSD requirements need to be defined.

<End of Text Proposal>