3GPP TSG-RAN WG4 Meeting #97-e                                                         R4-2014128

Online, 2nd-13th Nov, 2020

Source:	Samsung, TELUS, Bell mobility

Title:	TP for TR 37.717-21-11: DC_5A-7A_n66A

Agenda item:			10.4.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_5A-7A_n66A according to the request in [1].

# 2. Reference

- RP-201728, Revised WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.x	DC_5-7_n66

### 5.x.1	Configuration for DC

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_5A-7A_n66A | DC_5A_n66A DC_7A_n66A |

### 5.x.2	Co-existence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes. And it can be get:

- IMD3 of band 7 UL and band n66 UL falling to band 5 DL.

- IMD2 and IMD3 of band 5UL and band n66 UL falling to band 7 DL.

The MSD could reuse the value for CA_3A-7A-26A.

### 5.x.3	∆TIB and ∆RIB values

For DC_5-7_n66, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_5A-7A-66A.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7_n66 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7_n66 | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.x.4	REFSENS requirements

Table 5.x.4-1 shows the required MSD levels for the DC configuration.

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A-7A_n66A | 5 | 835 | 5 | 25 | 880 | 17.5 | IMD3 ¦fB7-2*fn66¦ |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | 7 | 2504 | 5 | 25 | 2624 | 29.01 | IMD2 ¦fB5+fn66¦ |
|  | n66 | 1777.5 | 5 | 25 | 2197.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

<End of Text Proposal>