3GPP TSG-RAN WG4 Meeting #97-e                                                         R4-2014130

Online, 2nd-13th Nov, 2020

Source:	Samsung, TELUS, Bell mobility

Title:	TP for TR 37.717-31-11: DC_2-5-7_n66 and DC_2-5-7-7_n66

Agenda item:			10.5.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_2-5-7_n66 and DC_2-5-7-7_n66 according to the request in [1].

# 2. Reference

- RP-201572, Revised WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL).

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_2-5-7_n66 and DC_2-5-7-7_n66

### 5.1.x.1	Configuration for DC

Table 5.1.x.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |

### 5.1.x.2	∆TIB and ∆RIB values

For DC_2-5-7_n66 and DC_2-5-7-7_n66, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_2A-5A-7A-66A.

Table 5.1.x.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 DC_2-5-7-7_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.x.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 DC_2-5-7-7_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.1.x.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

<End of Text Proposal>