3GPP TSG-RAN WG4 Meeting # 97-e                                     R4-2014186

# Electronic Meeting, 2– 13 November, 2020

Agenda Item:	10.21.2.1

Source: 	MediaTek Inc.,

Title: 	REFSENS of n8 and n71 for 35MHz channel bandwidth

Document for:	Approval

Introduction

In RAN4 96-e meeting, R4-2011793 [1] about new channel bandwidths per band was approved. It proposes to add 35MHz Channel Bandwidth (CBW) for bands n8 and n71. In [1], it was indicated n8 and n71 de-sense for new channel bandwidths should be studied.

In this contribution, for solving de-sense issue, reference sensitivity of bands n8 and n71 for 35MHz CBW is proposed.

Discussion

Analysis of Proposed n8 and n71 REFSENS Levels

The entire bandwidth of band n8 or n71 is 35MHz. By adding new CBW of 35MHz for bands n8 and n71, the impact of TX excess noise and IMD3 noise in RX frequency region is significant and causes RX de-sense. In [1] the uplink (UL) configuration parameters are defined to be not scalable with BW for 35 MHz CBW in bands n8 and n71. In this paper, the RB configuration for 35MHz channel bandwidth is used as shown in Table 1 for defining reference sensitivity requirement.

Table 1: Uplink configuration of n8 and n71 for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |  |
| n8 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | 25 | 25 | 20 | 20 |  |  | 20 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 12 | 10 | 10 |  |  | 10 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: 1 Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Due to wide channel bandwidth of 35MHz, TX IMD3 noise of transmission signal and carrier leakage could fall into the RX. The IMD3 noise is much larger than TX excess noise and higher-order ACLR noise. Based on the measurement and evaluation of TX in RX noise, the proposed reference sensitivity relaxation of n8 and n71 are listed in Table 2 and 3.

Figure 1

Figure 2

Table 2: NR band n71 reference sensitivity relaxation for CBW of 35MHz

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 12 | dB |
| PCB isolation from TX to RX | 65 | dB |
| Primary RX front-end and path loss | 4.5 | dB |
| Diversity RX front-end and path loss | 4 | dB |
| Conductive rejection from PA output to LNA input | 57 | dB |
| SNR requirement for QPSK | -1 | dB |
| Baseline reference sensitivity without TX-noise impact | -88.6 | dBm/35MHz |
| REFSENS relaxation with respect to baseline | 18.0 | dB |

Table 3: NR band n8 reference sensitivity relaxation for CBW of 35MHz

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 12 | dB |
| PCB isolation from TX to RX | 65 | dB |
| Primary RX front-end and path loss | 4.5 | dB |
| Diversity RX front-end and path loss | 4 | dB |
| Conductive rejection from PA output to LNA input | 55 | dB |
| SNR requirement for QPSK | -1 | dB |
| Baseline reference sensitivity without TX-noise impact | -88.4 | dBm/35MHz |
| REFSENS relaxation with respect to baseline | 19.2 | dB |

According to the results in Table 2 and 3, the reference sensitivity requirement of n8 and n71 are provided in Table 4.

Proposal 1: The REFSENS requirement of n8 and n71 for 35MHz channel bandwidth are provided in Table 4.

Table 4: n8 and n71 of two antenna port reference sensitivity level

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |  |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -69.2 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -69.5 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -70.5 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -70.8 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

In terms of testing purpose, UL resource blocks are located as close as possible to downlink band which is the adopted testing scenario. However, due to wide channel bandwidth, there might be non-testing scenario that TX is at the left-hand side and the TX CIM3 could also fall into RX which is illustrated in Figure 3. Although the TX CIM3 noise is not severe but it’s impact to RX could not be ignored. The TX CIM3 value is smaller than TX IMD3 value but larger than TX excess noise. As for the non-testing scenario, it’s sensitivity level of 35MHz CBW may be better than testing scenario’s sensitivity level

Figure 3

Observation 1: As for scenario illustrated in Figure3, the REFSENS degradation due to TX noise may not be severe. Large MSD level may not be needed.

Proposal 2: Whether to study for defining additional REFSENS requirement of bands n8 and n71 of 35MHz channel bandwidth for additional scenario (CIM3, TX excess noise and higher-order ACLR) shown in Figure 3 shall be decided.

Conclusion

This contribution provides an analysis of REFSENS requirement for the new 35MHz channel bandwidth for NR bands n8 and n71. Due to the wide channel bandwidth, TX noise desensitizes UE receiver chain. Therefore, REFSENS requirements for 35MHz CBW are proposed as follows.

Proposal 1: The REFSENS requirement of n8 and n71 for 35MHz channel bandwidth are provided in Table 4.

Table 4: n8 and n71 of two antenna port reference sensitivity level

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |  |
| n8 | 15 | -97 | -93.8 | -91.4 | -85.8 |  |  | -69.2 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.1 | -91.7 | -87.2 |  |  | -69.5 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n71 | 15 | -97.2 | -94 | -91.6 | -86 |  |  | -70.5 |  |  |  |  |  |  |  | FDD |
|  | 30 |  | -94.3 | -91.9 | -87.4 |  |  | -70.8 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation 1: As for scenario illustrated in Figure3, the REFSENS degradation due to TX noise may not be severe. Large MSD level may not be needed.

Proposal 2: Whether to study for defining additional REFSENS requirement of bands n8 and n71 of 35MHz channel bandwidth for additional scenario (CIM3, TX excess noise and higher-order ACLR) shown in Figure 3 shall be decided.

- 4. References

[1]	R4-2011793, “WF on UE RF requirement (for 35 and 45 MHz channel bandwidths)”, Skyworks Solutions Inc., ZTE, Huawei, Qualcomm, RAN4 96-e meeting.