3GPP TSG-RAN4 Meeting #97-e	R4-2014220

# Online, 2-13 Nov., 2020

Agenda item:	7.15.1

Source:	Apple

Title:		On HST intra-frequency measurement requirements

Document for:	Discussion

# Introduction

Core part of R16 NR support for high speed train scenario has been completed. However, we found some ambiguity on intra-frequency measurement requirement. In this contribution, we will elaborate the issue and propose some candidate solution to address it.

# Discussion

During core part design, RRM requirements for high speed scenario in connected state were introduced in intra-frequency RRM requirements, including cell identification, time index detection and measurement requirements. The enhanced requirements cover both intra-frequency measurement with and without measurement gap.

For intra-frequency measurement with gap, enhanced requirements are specified in TS38.133 clause 9.6.2.

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

Table 9.2.6.2-3: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 3 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(M2Note 1x 3) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 3 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When RRM enhancement for high speed is not configured, M2 = 1.5; When RRM enhancement for high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

Table 9.2.6.3-3: Measurement period When RRM enhancement for high speed is configured (Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x max(MGRP, SMTC period)) Note 1 |
| DRX cycle≤ 160ms | max(200ms, ceil(M2Note 2 x 5) x max(MGRP, SMTC period,DRX cycle)) |
| 160ms < DRX cycle≤ 320ms | max(200ms, ceil(M2Note 2 x 4) x max(MGRP, DRX cycle)) |
| DRX cycle>320ms | Y Note 3 x max(MGRP, DRX cycle) |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

As can be seen in above tables, MGRP is taken into account in high speed requirements. Measurement gap can be configured for two purposes. One is for intra-frequency measurement if the target SSB is not confined within UE active BWP. The other one is for inter-frequency measurement. Even though inter-frequency measurement was not enhanced, it was not precluded from network configuration point of view according to current specification. If both gap based intra-frequency measurement and inter-frequency measurement are configured, the measurement gaps have to be shared between intra and inter-frequency measurement. From requirements perspective this is reflected in the scaling factor CSSF. However, the scaling factor CSSF in high speed measurement period requirement is missing.

Observation 1: scaling factor CSSF is missing in high speed measurement period requirement.

There are two candidate options to address this issue:

Option 1: introduce CSSF in high speed measurement period requirement

Option 2: clarify in the specification that network shall not configure inter-frequency measurement for the UE when high speed is configured.

Both options can work. But option 2 would bring some unnecessary restriction in terms of network deployment and configuration. While in option 1 there is no such restriction. On the other hand, option 1 won’t increase any complexity since whether to configure inter-frequency measurement is still under network control.

Proposal 1: introduce CSSF in high speed measurement period requirement.

# Conclusion

In this contribution, we identify some issue in intra-frequency measurement requirement in high speed scenario. After discussion the following conclusions are provided:

Observation 1: scaling factor CSSF is missing in high speed measurement period requirement.

Proposal 1: introduce CSSF in high speed measurement period requirement.

# References

- TS38.133-g50