3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014244

# Electronic Meeting, 2 – 13 Nov, 2020

Agenda item:	7.9.2

Source:	Apple Inc.

Title:	Discussion on RRM requirements for Multi-TRP

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#96e the core requirements for eMIMO were concluded and way forward [1] was approved. In this contribution we discuss some of the assumptions and agreements made in RAN4#96e.

# 2. Discussion

In [1] it was agreed for Rel-16 eMIMO multi-TRxP transmission:

- No RRM core requirement impact identified on MRTD/MTTD values specified in Rel-15;

- It is RAN4 common understanding that MRTD/MTTD requirements in clauses 7.5.3, 7.6.3 and 7.6.4 is sufficient for support the deployment with multi-DCI based and single-DCI based multi-TRxP transmission.

We also have the following agreement in RAN4 RRM chairman report:

Agreement: UE may assume that all signals from multi-TRxPs of the same serving cell will be received within CP in intra-band contiguous CA scenarios

The agreement above is specifically for intra-band contiguous CA scenarios where we don’t have MRTD requirements. The agreement clearly captures that signals from multi-TRxPs from the same serving cell will be received within CP but doesn’t cover the case of multiple CCs. Although it is common understanding RAN4 that signals from different CCs in intra-band contiguous CA are received within a fraction of the CP length, the agreement above doesn’t capture the case of signals from multi-TRxP from multiple CCs. In order to add further clarity, we propose to update section 7.6.4 in TS 38.133 for MRTD requirements for NR-CA to capture the proposal.

Proposal #1: Update in MRTD requirements for NR CA that UE may assume that all signals from all CCs and multi-TRxP will be received within CP in intra-band contiguous CA scenario.

Our proposal is captured in our CR [2].

# 3. Conclusion

In this contribution we discuss some of the assumptions and agreements made in RAN4#96e for RRM requirements with multi-TRxP. Our proposal is captured below:

Proposal #1: Update in MRTD requirements for NR CA that UE may assume that all signals from all CCs and multi-TRxP will be received within CP in intra-band contiguous CA scenario.

# Reference

- R4-2012146, “WF on Completing Rel-16 eMIMO RRM Core Requirement”, Samsung.

- R4-2014245, “CR to 38.133 on RRM requirements for multi-TRxP”, Apple.