3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014247

# Electronic Meeting, 2 – 13 Nov, 2020

Agenda item:	7.9.3.2

Source:	Apple Inc.

Title:	Simulation results for L1-SINR Measurement accuracy

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#93 the simulation assumptions for L1-SINR accuracy evaluation were agreed. In this contribution we provide simulation results for L1-SINR measurement accuracy.

# 2. Discussion

For CMR only L1-SINR measurement, the simulation assumptions agreed in [1] are captured in the table below.

Table 1: Simulation assumption for L1-SINR accuracy evaluation for CMR only

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | N/A |
| Side condition (SNR) on CMR | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1, 3, 5 |
| Number of PRBs | 48 |
| Propagation condition | AWGN |

For CMR only measurement, we provide the measurement accuracy result in Table 2 below.

Table 2: Simulation results for CMR only based L1-SINR measurement accuracy

| Meas Error (dB) | Num Samples | Mean | 5th perc | 95th perc | Max |
| --- | --- | --- | --- | --- | --- |
| 15KHz | 1 | 1.08 | -0.59 | 2.81 | 2.81 |
|  | 3 | 1.02 | -0.06 | 2.44 | 2.44 |
|  | 5 | 1.01 | 0.10 | 2.40 | 2.40 |
| 30KHz | 1 | 1.09 | -0.63 | 2.88 | 2.88 |
|  | 3 | 1.03 | -0.08 | 2.52 | 2.52 |
|  | 5 | 1.02 | 0.02 | 2.42 | 2.42 |
| 120KHz | 1 | 1.10 | -0.54 | 2.86 | 2.86 |
|  | 3 | 1.04 | -0.07 | 2.53 | 2.53 |
|  | 5 | 1.03 | 0.04 | 2.44 | 2.44 |

Based on the results, we observe that there is no significant improvement in measurement accuracy with 3 vs 5 samples. Also, the delta with 3 samples averaging compared to no averaging is less than 0.5 dB across all cases and hence the accuracy requirements can be defined based on performance of single shot measurement.

Proposal #1: Define measurement accuracy for CMR based L1-SINR based on results from single shot measurement

Based on the results, the measurement accuracy is ± 3dB in both FR1 and FR2. Drawing from the principle used in defining requirements for L1-RSRP measurement accuracy in Rel-15, we propose to add an implementation margin of 2 dB in FR1 and 3.5 dB in FR2.  Hence, we propose to define the measurement accuracy for CMR based L1-SINR measurement as ±5 dB in FR1 and ±6.5 dB in FR2.

Proposal #2: Define measurement accuracy requirement for CMR based L1-SINR measurement as ±5 dB in FR1 and ±6.5 dB in FR2.

# 3. Conclusion

In this paper, we provide simulation results for L1-SINR acuuracy evaluation. Our proposals are summarized below:

Proposal #1: Define measurement accuracy for CMR based L1-SINR based on results from single shot measurement

Proposal #2: Define measurement accuracy requirement for CMR based L1-SINR measurement as ±5 dB in FR1 and ±6.5 dB in FR2.

# Reference

- R4-1915850, “Way forward on NR eMIMO RRM”, Samsung.