3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2014253

# Electronic Meeting, 2 – 13 Nov, 2020

Agenda item:	7.16.1.5

Source:	Apple Inc.

Title:	On Release Independence for NR UE performance enhancement requirements

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#96e release independence for UE performance enhancement requirements was discussed and way forward [1] was approved. In this contribution we present our views on release independence aspects.

# 2. Discussion

In [1] the options for defining requirements for PMI reporting with Type II codebook and CA/EN-DC power imbalance requirements are:

- PMI reporting requirements for Rel-15 type II codebook

  - Option 1: Release independent from Rel-15 (Ericsson, Huawei, China Telecom, CMCC)

  - Option 2: Not release independent from Rel-15 (Qualcomm, Apple)

- Issue 1-2: Release independent issue for CA/EN-DC power imbalance requirements

  - Option 1: Release independent from Rel-15 (DCM)

  - Option 2: Not release independent from Rel-15

The test setup for PMI reporting with Type II codebook is still under discussion in RAN4. In case MU-MIMO setup is agreed to be used, the test metric and requirements will be very different from what is used in Rel-15 PMI reporting tests.

Observation #1: Test setup for PMI reporting with Type II is still under discussion in RAN4

Observation #2: With MU-MIMO test setup the test metric for PMI reporting with Type II would be different than what is used in Rel-15 PMI reporting tests

The UE might not meet the requirements although it indicates capability for Type II codebook since it was not tested during development with the new requirements and testing criteria. Without consensus on the test setup, we cannot define PMI reporting requirements with Type II PMI as release independent since Rel-15.

Proposal #1: Do not define PMI reporting requirements with Type II codebook as release independent from Rel-15.

# 3. Conclusion

In this paper, we provide our views on release independence issues for PMI reporting tests with Type II codebook. Our observations and proposals are captured below:

Observation #1: Test setup for PMI reporting with Type II is still under discussion in RAN4

Observation #2: With MU-MIMO test setup the test metric for PMI reporting with Type II would be different than what is used in Rel-15 PMI reporting tests

Proposal #1: Do not define PMI reporting requirements with Type II codebook as release independent from Rel-15.

# Reference

- R4-2012687, “Way forward on release independence aspects for NR Rel-16 Type II PMI and power imbalance”, Huawei, HiSilicon.